Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

1 of 95

Table of Contents

APPENDIX A - NSFI SYSTEM NUMBER CODES .......................................................... 4

A.1 SUMMARY CHART....................................................................................................... 4

A.2 NSFI SUB-GROUPS FOR CODING OF DOCUMENTS AND ASSETS ........................................... 5

A.2.1 Specific requirements to NSFI coding of documents.....................................................6

A.2.2 Guideline for allocating NSFI codes to documents and drawings ...................................7

A.3 NSFI INTRODUCTION................................................................................................. 9

Main Group 1 - Rig General................................................................................................9

Group 10 – Spec., estim., model testing, draw., ordering, instruction material/courses .........................................9

Group 11 – Insurances, fees, general expenses, representation..............................................................................9

Group 12 – Quality assurance - General work and models....................................................................................9

Group 13 – Provisional rigging during construction (staging etc.) ...................................................................... 10

Group 14 – Work in connection with ways, launching and docking...................................................................... 10

Group 15 – Quality control, inspection, measurement, tests and trials................................................................. 10

Group 16 – Guarantee and mending work............................................................................................................. 10

Group 17 – Services to drilling rig during repair.................................................................................................. 11

Group 19 – General consumption articles............................................................................................................. 11

Main Group 2 - Hull and Structure .................................................................................... 11

Group 20 – Hull materials, general hull work....................................................................................................... 11

Group 21 – Pontoons............................................................................................................................................. 12

Group 22 – Columns, legs...................................................................................................................................... 12

Group 23 – Trussed frames.................................................................................................................................... 12

Group 24 – Main deck ........................................................................................................................................... 12

Group 25 – Superstructure and deck houses ......................................................................................................... 12

Group 26 – Hull outfitting ..................................................................................................................................... 13

Group 27 – Material protection, external.............................................................................................................. 13

Group 28 – Material protection, internal .............................................................................................................. 13

Main Group 3 - Drilling Equipment and Systems................................................................. 14

Group 30 – Derrick with components.................................................................................................................... 14

Group 31 – Drill floor equipment and systems...................................................................................................... 14

Group 32 – Bulk and Mud Systems incl. Cement, Brine & Base ........................................................................... 14

Group 33 – Well control equipment and systems................................................................................................... 15

Group 34 – Pipe handling equipment and systems................................................................................................ 15

Group 35 – Drill string and downhole equipment ................................................................................................. 15

Group 36 – Material handling equipment and systems ......................................................................................... 16

Group 37 – Service equipment and systems........................................................................................................... 16

Group 38 – Misc. equipment, systems and services............................................................................................... 16

Main Group 4 - Platform Equipment.................................................................................. 17

Group 40 – Manoeuvring machinery and equipment ............................................................................................ 17

Group 41 – Navigation and searching equipment ................................................................................................. 17

Group 42 – Communication equipment ................................................................................................................. 17

Group 43 – Anchoring, mooring and towing equipment........................................................................................ 18

Group 44 – Repair, maintenance and cleaning equipment, outfitting ................................................................... 18

Group 45 – Lifting and transport equipment for machinery components.............................................................. 18

Group 48 – Special equipment............................................................................................................................... 18

Main Group 5 - Equipment for Crew.................................................................................. 19

Group 50 – Lifesaving, protection and medical equipment ................................................................................... 19

Group 51 – Insul., panels, bulkheads, doors, side scuttles, windows, skylights..................................................... 19

Group 52 – Internal decks covering, ladders, steps, railing etc. ........................................................................... 19

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

2 of 95

Group 53 – External deck covering, steps, ladders etc., fore-and-aft gangway .................................................... 19

Group 54 – Furniture, inventory and entertainment equipment ............................................................................ 20

Group 55 – Galley & pantry equip., arrangement for provision, ironing/drying equipment ................................ 20

Group 56 – Lifting and transport equipment for crew and provisions .................................................................. 20

Group 57 – Ventilation, air-conditioning and heating systems ............................................................................. 20

Group 58 – Sanitary systems with discharges ....................................................................................................... 21

Main Group 6 - Machinery Main Components..................................................................... 21

Group 62 – Other types of propulsion machinery.................................................................................................. 21

Group 63 – Transmission and foils........................................................................................................................ 21

Group 64 – Boilers, steam and gas generators...................................................................................................... 21

Group 65 – Motor aggregates for main electric power production....................................................................... 22

Group 66 – Other aggregates & generators for main & emergency electrical power production........................ 22

Main Group 7 - Systems for Machinery Main Components................................................... 22

Group 70 – Fuel systems........................................................................................................................................ 22

Group 71 – Lube oil systems.................................................................................................................................. 22

Group 72 – Cooling systems.................................................................................................................................. 22

Group 73 – Compressed air systems...................................................................................................................... 23

Group 74 – Exhaust systems and air intakes ......................................................................................................... 23

Group 76 – Distilled and make-up water systems.................................................................................................. 23

Group 79 – Automation systems for machinery..................................................................................................... 23

Main Group 8 - Platform Common Systems........................................................................ 24

Group 80 – Ballast and bilge systems, gutter pipes outside accommodation ........................................................ 24

Group 81 – Fire & lifeboat alarm, fire- fighting & wash down systems ............................................................... 24

Group 82 – Air and sounding systems from tanks to deck ..................................................................................... 24

Group 83 – Special common hydraulic oil systems ............................................................................................... 24

Group 84 – Central heat transfer systems with chemical liquids .......................................................................... 24

Group 85 – Automation common systems.............................................................................................................. 25

Group 86 – Electrical power supply...................................................................................................................... 25

Group 87 – Electrical distribution common systems ............................................................................................. 25

A.4 NSFI DETAIL DESCRIPTION AND GUIDE....................................................................... 26

Main Group 2 - Hull and Structure .................................................................................... 26

Group 26 – Hull outfitting ..................................................................................................................................... 26

Main Group 3 - Drilling Equipment and Systems................................................................. 27

Group 30 – Derrick with Components................................................................................................................... 27

Group 31 – Drill floor equipment and systems...................................................................................................... 29

Group 32 – Bulk and Mud Systems incl. Cement, Brine & Base ........................................................................... 31

Group 33 – Well control equipment and systems................................................................................................... 35

Group 34 – Pipe handling equipment and systems................................................................................................ 38

Group 35 – Drill string and downhole equipment ................................................................................................. 40

Group 36 – Material handling equipment and systems ......................................................................................... 41

Group 37 – Service equipment and systems........................................................................................................... 43

Group 38 – Misc. equipment, systems and services............................................................................................... 46

Main Group 4 - Platform Equipment.................................................................................. 48

Group 40 – Manoeuvring machinery and equipment ........................................................................................... 48

Group 41 – Navigation and searching equipment ................................................................................................. 49

Group 42 – Communication equipment ................................................................................................................. 51

Group 43 – Anchoring, mooring and towing equipment........................................................................................ 53

Group 44 – Repair, maintenance and cleaning equipment, outfitting ................................................................... 55

Group 45 – Lifting and transport equipment for machinery components.............................................................. 57

Group 48 – Special equipment............................................................................................................................... 58

Main Group 5 - Equipment for Crew.................................................................................. 59

Group 50 – Lifesaving, protection and medical equipment ................................................................................... 59

Group 51 – Insul., panels, bulkheads, doors, side scuttles, windows, skylights..................................................... 60

Group 52 – Internal decks covering, ladders, steps, railing etc. ........................................................................... 61

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

3 of 95

Group 53 – External deck covering, steps, ladders etc., fore-and-aft gangway .................................................... 62

Group 54 – Furniture, inventory and entertainment equipment ............................................................................ 63

Group 55 – Galley & pantry equip., arrangement for provision, ironing/drying equipment ................................ 64

Group 56 – Lifting and transport equipment for crew and provisions .................................................................. 65

Group 57 – Ventilation, air-conditioning and heating systems ............................................................................. 66

Group 58 – Sanitary systems with discharges ....................................................................................................... 68

Main Group 6 - Machinery Main Components..................................................................... 69

Group 62 – Other types of propulsion machinery.................................................................................................. 69

Group 63 – Transmission and foils........................................................................................................................ 69

Group 64 – Boilers and Steam Plants.................................................................................................................... 70

Group 65 – Motor aggregates for main electric power production....................................................................... 70

Group 66 – Other aggregates & generators for main & emergency electrical power production........................ 71

Main Group 7 - Systems for Machinery main Components................................................... 72

Group 70 – Fuel systems........................................................................................................................................ 72

Group 71 – Lube oil systems.................................................................................................................................. 75

Group 72 – Cooling systems.................................................................................................................................. 78

Group 73 – Compressed air systems...................................................................................................................... 80

Group 74 – Exhaust systems and air intakes ......................................................................................................... 82

Group 76 – Distilled and make-up water systems.................................................................................................. 82

Group 79 – Automation systems for machinery..................................................................................................... 83

Main Group 8 - Platform Common Systems........................................................................ 85

Group 80 – Ballast and bilge systems, gutter pipes outside accommodation ........................................................ 85

Group 81 – Fire & lifeboat alarm, fire- fighting & wash down systems ............................................................... 88

Group 82 – Air and sounding systems from tanks to deck ..................................................................................... 91

Group 83 – Special common hydraulic oil systems ............................................................................................... 91

Group 84 – Central heat transfer systems with chemical liquids .......................................................................... 92

Group 85 – Automation common systems.............................................................................................................. 92

Group 86 – Electrical power supply...................................................................................................................... 93

Group 87 – Electrical distribution common systems ............................................................................................. 94

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

4 of 95

APPENDIX A - NSFI SYSTEM NUMBER CODES

A.1 Summary Chart

1

Rig General

2 Hull and

Structure

3 Drilling

Equipment and

System

4 Platform

Equipment

5 Equipment for

crew

6 Machinery

Main

Components

7 Systems for

Machinery Main

Components

8 Platform

Common

Systems

1

1

0

SPEC., ESTIM., MODEL

TESTING, DRAW.,

ORDERING

INSTR.MAT./COURSES

INSURANCES, FEES,

GEN. EXPENSES,

20

HULL MATERIALS,

GENERAL HULL WORK

30

DERRICK WITH

COMPONENTS

40

MANOEUVRING

MACHINERY AND

EQUIPMENT

50

LIFESAVING,

PROTECTION AND

MEDICAL EQUIPMENT

70

FUEL SYSTEMS

80

BALLAST AND BILGE

SYSTEMS, GUTTER

PIPES OUTSIDE

ACCOMMODATION

FIRE & LIFEBOAT

ALARM, FIRE-

1

21

PONTOONS

31

DRILL FLOOR

EQUIPMENT AND

SYSTEMS

41

NAVIGATION AND

SEARCHING

EQUIPMENT

51

INSUL., PANELS,

BULKHEADS, DOORS,

SIDE SCUTTLES,

WINDOWS,

71

LUBE OIL SYSTEMS

81

REPRESENTATION

FIGHTING & WASH

DOWN SYSTEMS

SKYLIGHTS

1

1

1

2

3

4

QUALITY ASSURANCE

22

23

24

COLUMNS, LEGS

TRUSSED FRAMES

MAIN DECK

32

33

34

BULK AND MUD

SYSTEMS INCL.

CEMENT, BRINE &

BASE

WELL CONTROL

EQUIPMENT AND

SYSTEMS

42

43

44

COMMUNICATION

EQUIPMENT

52

53

54

INTERNAL DECK

COVERING, LADDERS,

STEPS, RAILING, ETC.

62

63

64

OTHER TYPES OF

PROPULSION

MACHINERY

72

73

74

COOLING SYSTEM

82

83

84

AIR AND SOUNDING

SYSTEMS FROM

TANKS TO DECK

–

GENERAL WORK AND

MODELS

PROVISIONAL

ANCHORING

MOORING AND

TOWING EQUIPMENT

EXTERNAL DECK

COVERING, STEPS,

LADDERS, ETC, FORE-

AND-AFT GANGWAY

FURNITURE,

INVENTORY AND

ENTERTAINMENT

EQUIPMENT

TRANSMISSIONS

AND FOILS

COMPRESSED AIR

SYSTEMS

SPECIAL COMMON

HYDRAULIC OIL

SYSTEMS

RIGGING DURING

CONSTRUCTION

(STAGING ETC.)

WORK IN

CONNECTION WITH

WAYS, LAUNCHING

AND DOCKING

PIPE HANDLING

EQUIPMENT AND

SYSTEMS

REPAIR,

MAINTENANCE AND

CLEANING

EQUIPMENT,

OUTFITTING

LIFTING AND

TRANSPORT

EQUIPMENT FOR

MACHINERY

COMPONENTS

BOILERS, STEAM

AND GAS

GENERATORS

EXHAUST SYSTEM

AND AIR INTAKES

CENTRAL HEAT

TRANSFER SYSTEMS

WITH CHEMICAL

LIQUIDS

1

1

5

6

QUALITY CONTROL,

INSPECTION,

MEASUREMENTS,

TESTS AND TRIALS

25

26

SUPERSTRUCTURE

AND DECK HOUSES

35

36

DRILL STRING AND

DOWNHOLE

EQUIPMENT

45

55

56

GALLEY & PANTRY

EQUIPMENT ,

ARRANGEMENT FOR

PROVISIONS,

IRONING/DRYING

EQUIPMENT

LIFTING AND

TRANSPORT

EQUIPMENT FOR

CREW AND

PROVISIONS

65

66

MOTOR

85

86

AUTOMATION

COMMON SYSTEMS

AGGREGATES FOR

MAIN ELECTRIC

POWER

PRODUCTION

GUARANTEE AND

MENDING WORK

HULL OUT FITTING

MATERIAL HANDLING

EQUIPMENT AND

SYSTEMS

OTHER

76

DISTILLED AND

MAKE-UP WATER

SYSTEMS

ELECTRICAL

DISTRIBUTION

COMMON SYSTEMS

AGGREGATES &

GENERATORS

FOR MAIN &

EMERGENCY

ELECTRICAL

POWER

PRODUCTION

1

1

7

9

SERVICES TO

DRILLING RIG DURING

REPAIR

27

MATERIAL

PROTECTION,

EXTERNAL

MATERIAL

PROTECTION,

INTERNAL

37

38

SERVICE EQUIPMENT

AND SYSTEMS

57

58

VENTILATION, AIR-

CONDITIONING AND

HEATING SYSTEMS

SANITARY SYSTEMS

WITH DISCHARGES

2

8

MISC. EQUIPMENT,

SYSTEMS AND

SERVICES

48

SPECIAL EQUIPMENT

GENERAL

CONSUMPTION

ARTICLES

79

AUTOMATION

SYSTEMS FOR

MACHINERY

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

5 of 95

A.2 NSFI Sub-Groups for coding of documents and assets

Asset Tag Categories and Document Categories are defined as follows:

Asset Categories

Main Equipment and

Tag Code format

NSFI Sub-Group within a code (Bold)

Comments

1

NNN-AA-NNN(A)

NNN-AA-NNN(A)

1)

Equipment Packages

Equipment Packages

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

NNN-AA-NNN(A)

1)

1)

1)

1)

1)

1)

1)

Main Equipment (General)

Mechanical Equipment

HVAC Systems and Equipment

Main Electrical Equipment

Automation and Control Systems

Central telecommunication

2

Piping and Valves

Pipes and lines

NNNANNN

NNNN-AA-NNNANNN-AANNN(A)-NA

NNNANNN - AAAANNA

NNNANNN - AAAANNA

NNNANNN

2)

3)

3)

4)

3)

3)

3)

5)

6)

7)

8)

9)

Manual Valves

NNNANNN

Check Valves

NNNANNN

Hoses / Flexible Pipes

Safety Relief Valves

Control Valves

NNNANNN

NNN-AAA-NNNN

NNN-AAA-NNNN

NNN-AAA(A)-NNNN

AZZZZ-AAA-NNNN

NNN-A-AA(AAA)-NNNN(A)

NNN-A-AA-NNNN

NNN-A-AA-NNNN

NNN-A-AA-NNNN

NNN-AAA-NNNN

NNN-AAA-NNNN

Actuated Valves (On/Off)

Pipe Supports

NNN-AAA(A)-NNNN - AAAANNA

AZZZZ-AAA-NNNN

3

4

5

6

Instrument Field Equipment

Fire and Gas

NNN-A-AA(AAA)-NNNN(A)

NNN-A-AA-NNNN

Telecommunication Appliances

Junction Boxes

NNN-A-AA-NNNN

NNN-A-AA-NNNN

Electrical Lighting Sockets and

Switches

7

NNN-A-AA-AANNN-NNN-NNN

NNN-A-AA-A-NNNN(A)

NNN-A-AA-AANNN-NNN-NNN

NNN-A-AA-A-NNNN(A)

10)

11)

8

9

Cables

Switching Devices

Switching devices within

Main Switchboards/MCCs

NNN-AA-NNN-NNZZ-A-NNN

NNN-AA-NNN-A-NNN

NNN-AA-NNN-NNZZ-A-NNN

NNN-AA-NNN-A-NNN

12)

12)

Switching devices within

Distribution Boards/Panels

1

0

Special Items

NNNANNN-AANNN

NNNANNN- AANNN

13)

Documents

Document Code format

NSFI Sub-Group within a code (Bold)

Comments

Drawings and documents

NNN-NNN-A-AA-NNN

NNN-NNN-A-AA-NNN

14)

14)

Drawings and document with sheet

number allocated

NNN-NNN-A-AA-NNN.NN

NNN-NNN-A-AA-NNN .NN

Comments:

1

2

3

4

5

)

See the detailed description of substructure for relation to actual the NSFI Sub-Group.

Shall follow the same NSFI Sub-Group as the Main Equipment / Equipment Package it`s provide.

Shall follow the same NSFI Sub-Group as the piping system it`s installed within.

Shall follow the same NSFI Sub-Group as the provided equipment.

NSFI coding not applicable.

)

)

)

)

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

6 of 95

6

7

8

9

1

1

1

1

1

)

)

)

)

Shall follow the same NSFI Sub-Group as the parent tag.

Fire and Gas system NSFI Sub-Group 811.

Shall follow NSFI Sub-Group for equipment within Main Group 4.

Shall follow the same NSFI Sub-Group as consumers in the circuit.

0) Shall follow the same NSFI Sub-Group as the feeder Switchboard/Distribution Board.

1) Shall follow the same NSFI Sub-Group as cable consumer.

2) NSFI coding is part of Main Switchboard/Distribution board main equipment code.

3) Special Items shall follow the same NSFI Sub-Group as the system it`s installed within.

4) NSFI coding according to Appendix A.2.1 and A.2.2

The NSFI Group System is built up as a 3-digit decimal classification system. The rig is divided into

10 main Groups from 0 to 9. Only main Groups 1 to 8 are in use.

Main Group 1 - Rig General

Main Group 2 - Hull and Structure

Main Group 3 – Drilling Equipment and Systems

Main Group 4 - Platform Equipment

Main Group 5 - Equipment for Crew

Main Group 6 - Machinery Main Components

Main Group 7 - Systems for Machinery main Components

Main Group 8 - Platform Common Systems

An example of subdivision (Sub Group System) of Main Group 7 is given below:

Main Group

Group System

7

73

System for Machinery Main Components

Compressed Air Systems

Sub Group System

731

Starting Air Systems

A.2.1 Specific requirements to NSFI coding of documents

Drawings can easily be numbered according to NSFI. Documents and drawings should be

numbered according to Sub-Group level if possible. Main Group levels are usually used for General

Arrangement and Layout/Location drawings and Structural drawings.

Used when not possible to place a document in a Sub-Group.

Examples of Main Group format:

1

00

Rig General

Hull and Structure

200

Examples of Group format:

7

8

30

50

Compressed Air Systems

Electrical Systems general parts

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

7 of 95

A.2.2 Guideline for allocating NSFI codes to documents and drawings

Rig General - Multidiscipline

NSFI Main

Group

Document

Type Codes

Document Type Description

Comments

1

00

1

1

1

00

00

00

XD

XE

XF

General Arrangements (GA) – Rig General

Layout drawings – Rig General

Location drawings (plot plans) – Rig General

Main Group level

Main Group level

Main Group level

Common Sub-Group Level

NSFI Sub-

Group

Document

Type Codes

Document Type Description

Comments

1

01

1

1

1

1

01

01

01

01

CA

LF

RE

RF

Analysis, test and calculations

Legends and typical

DFI (Design-Fabrication-Installation) resumes

Verification reports

Common Sub-Group Level

Common Sub-Group Level

Common Sub-Group Level

Common Sub-Group Level. If related to

specific Sub-Group, use actual NSFI Sub-

Group.

1

1

1

01

01

01

SA

SP

TF

Project Specifications

Project Standards

Technical Folder / Drawing File

Common Sub-Group Level

Common Sub-Group Level

Common Sub-Group Level. If related to

specific Sub-Group, use actual NSFI Sub-

Group.

1

1

1

1

1

01

01

01

01

01

WA

WD

WH

WL

XP

Containment (Inspection manuals)

DROPS (Inspection manuals)

Hull and structure (Inspection manuals)

Drilling and lifting equipment (Inspection manuals)

Instruction and information plans

Common Sub-Group Level

Common Sub-Group Level

Common Sub-Group Level

Common Sub-Group Level

Common Sub-Group Level

Certificates – Main rig

NSFI Sub-

Group

Document

Type Codes

Document Type Description

Comments

1

12

1

12

VB

Certificates

Specific Sub-Group Level

Common Hull/Structural/Steel Main Group level

NSFI Sub-

Group

Document

Type Codes

Document Type Description

Comments

2

00

2

2

00

00

XG

XS

Structural information; included main structural steel,

secondary/outfitting steel, structural fire protection

and acoustic/thermal insulation and fire protection.

Detail cross sectional drawings

Main Group level

Main Group level

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

8 of 95

Specific Sub-Group level

NSFI Sub-

Group

Document

Type Codes

Document Type Description

Comments

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

DS

FD

LA

LC

LD

KA

IR

Data Sheets

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level. If not possible

to place in a sub-group, use NSFI Main

Group system or Group system.

Technical Functional Description

Lists, registers and indexes

User manual- / MRB content lists

LCI Supplier master document/information registers

Procedures

Inspection & Certification Reports

Sub-Group

MA

Installation-/Operation-/Maintenance Data for

Equipment User Manual

Specific Sub-Group Level

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

MB

MC

MG

MH

RB

Operation and maintenance instructions

Spare Parts List and Interchangeability Register (SPIR)

Equipment handling instructions

Lubrication schedules

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level. If not possible

to place in a sub-group, use NSFI Main

Group system or Group system.

If not related to specific Sub-Group, use

actual common NSFI Sub-Group.

Specific Sub-Group Level. If not possible

to place in a sub-group, use NSFI Main

Group system or Group system.

Specific Sub-Group Level

Modification Notice/package

Sub-Group

Sub-Group

RA

RD

Reports

System design reports and system user manuals

Sub-Group

Sub-Group

Sub-Group

Sub-Group

TA

TB

TP

VA

Plans and schedules

Work plans

Work Package

Specific Sub-Group Level

Specific Sub-Group Level

Manufacturing Record Book (MRB)

Specific Sub-Group Level. If not possible

to place in a sub-group, use NSFI Main

Group system or Group system.

Specific Sub-Group Level. For Main Rig

Certificates, see NSFI Sub Group 112.

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level

Sub-Group

VB

Certificates

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

Sub-Group

WM

XA

XB

XD

XE

XF

XI

XJ

XK

XL

Mechanical completion and commissioning dossiers

Flow diagrams

Pipe and instrument diagrams (P&ID)

General Arrangements (GA)

Layout drawings

Location drawings (plot ) plans

System topology and block diagrams

Single line diagrams

Circuit diagrams

Logic diagrams

Level diagrams

Pipe supports

XM

XO

XR

XT

Cause and effect diagram

Wiring diagrams/Termination diagram for external

connection

Specific Sub-Group Level

Sub-Group

Sub-Group

Sub-Group

XU

XW

XX

Loop diagrams

Alignment sheet

Drawings - miscellaneous

Specific Sub-Group Level

Specific Sub-Group Level

Specific Sub-Group Level. If not possible

to place in a sub-group, use NSFI Main

Group system or Group system.

Specific Sub-Group Level. If not possible

to place in a sub-group, use NSFI Main

Group system or Group system.

If not possible to specific sub-group, use

related to specific Sub-Group, use actual

NSFI Sub-Group.

Sub-Group

Sub-Group

XY

XZ

Hook-up drawings

Inspection drawings

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

9 of 95

A.3 NSFI Introduction

Main Group 1 - Rig General

Group 10 – Spec., estim., model testing, draw., ordering, instruction material/courses

1

RIG GENERAL

SPEC., ESTIM., MODEL TESTING, DRAW., ORDERING INSTR.MAT./COURSES

10

1

1

1

1

1

1

1

1

1

01

Contract terms, spec., design studies, drawings, model testing

Drawing, ordering etc. w.r.t. hull, hull half model

02

03

04

05

06

07

08

09

Drawing, ordering etc. w.r.t. drilling equipment and systems

Drawing, ordering etc. w.r.t. misc. platform equipment

Drawing, ordering etc. w.r.t. equipment for crew

Drawing, ordering etc. / machinery main comp. & mach. / platform system

Estimating, drawing & quotations w.r.t. alteration work etc.

Purchase/renting of drawings, gen. patent/-license-/consult. fees

Instruction material, instruction courses

Group 11 – Insurances, fees, general expenses, representation

1

RIG GENERAL

INSURANCES, FEES, GEN. EXPENSES, REPRESENTATION

11

1

1

1

1

1

1

1

1

11

General Insurances

12

13

14

15

16

17

18

Classification and authorities etc. – fees and certificates

Interest on build advances, discount rate losses, financing

Cancellation expenses

Penalties

Broker's commissions

General representation and travelling, conferences

Representation at launching and delivery

Group 12 – Quality assurance - General work and models

1

RIG GENERAL

QUALITY ASSURANCE – GENERAL WORK AND MODELS

12

1

1

1

1

1

1

1

21

Quality assurance planning, work preparation and reporting

Guard, supervision and watch keeping

Clearing and cleaning

22

23

24

25

26

27

Snow cleaning and de-icing

Transportation (excl. hull materials and sections)

Assistance with owner's supply

Arr. models, full scale models, exhib. models, photogr./albums

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

10 of 95

Group 13 – Provisional rigging during construction (staging etc.)

1

RIG GENERAL

PROVISIONAL RIGGING DURING CONSTRUCTION (STAGING ETC.)

13

1

1

1

1

1

1

1

31

Prov. gangways, roofs, hatches, stairs, railing etc. for hull

Provisional hatches, ladders, railing and stages internal

Provisional electric power, lighting and machinery

Provisional steam, water, heating and vent. plants

Provisional gas, oxygen and compressed air plants

Tarpaulin rental

34

35

36

37

38

39

Provisional lifting and transport equipment

Group 14 – Work in connection with ways, launching and docking

1

RIG GENERAL

WORK IN CONNECTION WITH WAYS, LAUNCHING AND DOCKING

14

1

1

1

1

1

1

41

Arrangement of keel blocks and pillars

42

43

44

45

46

Launch. arr. with cradle, supports, water filling etc., launching platform

Crew, pilots, tugs, clearing in connection with launching

Dry docking, slip docking

Shifting and towing not in connection with launching

Gas freeing, tank cleaning

Group 15 – Quality control, inspection, measurement, tests and trials

1

RIG GENERAL

QUALITY CONTROL, INSPECTION, MEASUREMENTS, TESTS AND TRIALS

15

1

1

1

1

1

51

Machinery testing

52

53

54

55

Capacity, inclination, vibration measurements, etc.

Fuel and lube oil for tests and trips

Trials (trial trips)

Special tests/trials

Group 16 – Guarantee and mending work

1

RIG GENERAL

GUARANTEE AND MENDING WORK

16

1

1

1

1

61

Mending work after trials

62

64

65

Guarantee work including travelling

Repair of damages caused by yard

Suppliers and service companies

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

11 of 95

Group 17 – Services to drilling rig during repair

1

RIG GENERAL

SERVICES TO DRILLING RIG DURING REPAIR

17

1

1

1

1

1

71

Disposal of waste/garbage

72

73

74

75

Crane assistance to the drilling rig' s crew

Transport of yard personnel

Outlets from toilets and wash rooms

Services to the drilling rig's crew (meals, etc.)

Group 19 – General consumption articles

1

RIG GENERAL

GENERAL CONSUMPTION ARTICLES

19

1

1

1

1

1

1

1

1

1

91

Electrodes

92

93

94

95

96

97

98

99

Gas, oxygen, coal and oil

Tools and jigs

Binding and mending materials, cleaning articles

Equipment for owner's representatives

Miscellaneous consumption articles

Electrical Consumables

Steel (general consumption)

Pipe, hose, fittings and connections

Main Group 2 - Hull and Structure

Group 20 – Hull materials, general hull work

2

HULL AND STRUCTURE

HULL MATERIALS, GENERAL HULL WORK

20

2

2

2

2

2

01

Hull materials

02

03

04

05

Steel transportation, sorting and storage

Sandblasting, shop priming, straight rolling & cleaning material

Testing of tanks, bulkheads, etc.

X-ray and ultrasonic testing of hull parts

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

12 of 95

Group 21 – Pontoons

2

HULL AND STRUCTURE

PONTOONS

21

2

11

Shell plating, pontoons

Group 22 – Columns, legs

2

HULL AND STRUCTURE

COLUMNS, LEGS

22

2

2

2

21

Column shell

22

23

Connections / attachments

Chain lockers and trunks, etc.

Group 23 – Trussed frames

2

HULL AND STRUCTURE

TRUSSED FRAMES

Bracings

23

2

31

Group 24 – Main deck

2

HULL AND STRUCTURE

MAIN DECK

24

2

2

2

2

2

41

Main transverse girders and bulkheads

Main longitudinal girders and bulkheads

Main deck

42

43

44

45

Pipe racks for drill pipes and casings

Drill floor

Group 25 – Superstructure and deck houses

2

HULL AND STRUCTURE

SUPERSTRUCTURE AND DECK HOUSES

25

2

2

2

51

Superstructure

53

54

Deck houses

Small uninsulated houses, shelters

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

13 of 95

Group 26 – Hull outfitting

2

HULL AND STRUCTURE

HULL OUT FITTING

26

2

2

2

2

2

2

2

61

Name plates, draught marks, etc.

Drain-, bottom plugs, sea chests, manholes, watertight covers and hatches

Foundations

62

63

64

65

66

67

Boat bumpers, fender bars and wear bars

Safety shelters

Hawse pipes and chain pipes

Gutter bars, bulwark, breakwaters

Group 27 – Material protection, external

2

HULL AND STRUCTURE

MATERIAL PROTECTION, EXTERNAL

27

2

2

2

2

2

2

2

71

Lower parts of hull

72

73

74

75

76

78

Upper parts of hull

Weather decks

Equipment on weather decks

Deck houses

Galvanizing, nickelplating

Cathodic protection, external

Group 28 – Material protection, internal

2

HULL AND STRUCTURE

MATERIAL PROTECTION, INTERNAL

28

2

2

2

2

2

2

2

81

Rooms in accommodation, stores, box girder

Pump rooms, engine rooms, mud rooms etc.

Columns internal

82

83

85

87

88

89

Chain locker, ballast and seawater tanks

Fresh water tanks and misc. other tanks

Fuel- and lube oil tanks

Cathodic protection internal

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

14 of 95

Main Group 3 - Drilling Equipment and Systems

Group 30 – Derrick with components

3

DRILLING EQUIPMENT AND SYSTEMS

DERRICK WITH COMPONENTS

30

3

3

3

3

3

01

Derrick structure

02

03

04

05

Crown block with drill string motion compensator

Travelling equipment

Drill string heave compensator

Miscellaneous derrick systems

Group 31 – Drill floor equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

DRILL FLOOR EQUIPMENT AND SYSTEMS

31

3

3

3

3

3

3

3

11

Drilling control

12

13

14

15

16

17

Draw works & machinery

Rotary table, top drive and associated equipment

Tensioning systems

H.P. air system for tensioners and compensators

Wire lines

Miscellaneous drill floor equipment and systems

Group 32 – Bulk and Mud Systems incl. Cement, Brine & Base

3

DRILLING EQUIPMENT AND SYSTEMS

BULK AND MUD SYSTEMS INCL. CEMENT, BRINE & BASE

32

3

3

3

3

3

3

3

3

21

Bulk storage and transfer systems

Bulk air system

22

23

24

25

26

27

28

Mud additive system

Mud mixing and storage

Mud supply

Mud solids control

Drill water system

H.P. hoses, chicksans, piping and valves

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

15 of 95

Group 33 – Well control equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

WELL CONTROL EQUIPMENT AND SYSTEMS

33

3

3

3

3

3

3

3

31

Blowout preventer stack

32

33

34

35

36

37

Blowout preventer control system

Shallow gas stack with control system

Diverter with control systems

Riser systems incl. choke, kill and booster lines

Choke and kill systems incl. mud/gas separator

Miscellaneous well control related equipment

Group 34 – Pipe handling equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

PIPE HANDLING EQUIPMENT AND SYSTEMS

34

3

3

3

3

3

3

3

41

Derrick mounted vertical pipe handling system

Drill floor tubular handling equipment

Power packs for pipe/tubular handling

Slips and spiders

42

43

44

45

46

47

Elevators

Manual tongs

Miscellaneous equipment for pipe/tubular handling

Group 35 – Drill string and downhole equipment

3

DRILLING EQUIPMENT AND SYSTEMS

DRILL STRING AND DOWNHOLE EQUIPMENT

35

3

3

3

3

3

3

3

51

Drill pipes

52

53

54

55

56

57

Drill collars

Substitutes (tubular)

Stabilizers

Downhole drilling tools

Fishing tools

Misc. drill string and downhole equipment (incl. kelly)

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

16 of 95

Group 36 – Material handling equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

MATERIAL HANDLING EQUIPMENT AND SYSTEMS

36

3

3

3

3

3

61

Deck cranes

62

63

64

65

Overhead cranes

Monorail systems

Bop transportation system

Utility hoists

Group 37 – Service equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

SERVICE EQUIPMENT AND SYSTEMS

37

3

3

3

3

3

3

71

Cementing unit and associated system

Well testing equipment

72

73

74

75

76

Mud logging equipment

Electric well logging equipment

Special service equipment and supplies

Wellhead and tubular equipment

Group 38 – Misc. equipment, systems and services

3

DRILLING EQUIPMENT AND SYSTEMS

MISC. EQUIPMENT, SYSTEMS AND SERVICES

38

3

3

3

3

3

81

Wellhead and sub-sea running and testing tools

Sub-Sea inspection system

82

83

84

85

Acoustic position reference system

Miscellaneous instruments and auxiliary equipment

Hydraulic/pneumatic power packs (general purpose)

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

17 of 95

Main Group 4 - Platform Equipment

Group 40 – Manoeuvring machinery and equipment

4

PLATFORM EQUIPMENT

MANOEUVRING MACHINERY AND EQUIPMENT

40

4

4

4

4

01

Rudder with welded parts

03

06

08

Steering gear/-columns, telemotor system, rudder indicators

Brakes

Dynamical positioning plant (DP)

Group 41 – Navigation and searching equipment

4

PLATFORM EQUIPMENT

NAVIGATION AND SEARCHING EQUIPMENT

41

4

4

4

4

4

4

4

4

11

Radar Plant

12

13

14

15

16

17

18

Positions indicating systems

Gyro plants, autopilots, compasses

Underwater searching equipment: Asdic, echo sounder, speed log

Clinometers, trim indicators, load indicators, etc.

Navigation TV

Clocks, facsimile recorders and misc. nautical equipment

Radar, signal, observation and antenna masts

Group 42 – Communication equipment

4

PLATFORM EQUIPMENT

COMMUNICATION EQUIPMENT

42

4

4

4

4

4

4

21

Radio Plant

22

23

24

25

27

Lifeboat radio transmitters, emergency radio, direct. finder

Data transmission plants, general purpose EDB plants

VHF and UHF telephones etc.

Calling systems, command telephone, telephone plants, walkie-talkie, etc.

Light and signal equipment (lanterns, whistles, etc.)

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

18 of 95

Group 43 – Anchoring, mooring and towing equipment

4

PLATFORM EQUIPMENT

ANCHORING MOORING AND TOWING EQUIPMENT

43

4

4

4

4

4

4

4

31

Anchors with chains and equipment

Windlasses with chain stoppers, rollers, etc.

Combined windlass & mooring winches with chain stoppers, rollers

Capstans, warping and mooring winches

Fixed mooring equipment

32

33

34

35

36

37

Loose mooring equipment

Towing equipment

Group 44 – Repair, maintenance and cleaning equipment, outfitting

4

PLATFORM EQUIPMENT

REPAIR, MAINTENANCE AND CLEANING EQUIPMENT, OUTFITTING

44

4

4

4

4

4

4

4

4

41

Machine tools, cutting and welding equipment

Tools and equipment for engineers and electricians

Painting equipment and scaffolding, paint rafts/boats (gigs)

Cleaning equipment, garbage chutes, etc.

Garbage disposal plants

42

43

44

45

46

47

48

Outfitting in store rooms and workshops

Clamps and foundation for spare parts

Name plates (markings) on machinery, equipment, pipes, cables

Group 45 – Lifting and transport equipment for machinery components

4

PLATFORM EQUIPMENT

LIFTING AND TRANSPORT EQUIPMENT FOR MACHINERY COMPONENTS

45

4

4

4

4

51

Lifts

52

54

55

Travelling crane and lifting equipment in machinery rooms

Mobile transport and handling equipment

Miscellaneous Lifting Appliances (Loose Equipment)

Group 48 – Special equipment

4

PLATFORM EQUIPMENT

SPECIAL EQUIPMENT

48

4

4

4

4

83

Diving barge, total

85

87

89

Aircraft, helicopters

De-icing equipment

Derrick skidding systems

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

19 of 95

Main Group 5 - Equipment for Crew

Group 50 – Lifesaving, protection and medical equipment

5

EQUIPMENT FOR CREW

LIFESAVING, PROTECTION AND MEDICAL EQUIPMENT

50

5

5

5

5

5

01

Lifeboats with Equipment

02

03

04

05

Liferafts with Equipment

Lifesaving, safety and emergency equipment

Medical and dental equipment, medicines and first-aid equipment

Loose Fire-fighting apparatus and equipment, firemen’s suit

Group 51 – Insul., panels, bulkheads, doors, side scuttles, windows, skylights

5

EQUIPMENT FOR CREW

INSUL., PANELS, BULKHEADS, DOORS, SIDE SCUTTLES, WINDOWS, SKYLIGHTS

51

5

5

5

5

11

Insulation, partition bulkheads, panelling, wallpaper

Doors with coamings, etc., in accommodation

External doors with coamings, etc.

12

14

15

Side scuttles and windows with equipment

Group 52 – Internal decks covering, ladders, steps, railing etc.

5

EQUIPMENT FOR CREW

INTERNAL DECK COVERING, LADDERS, STEPS, RAILING, ETC.

52

5

5

5

5

5

5

21

Deck base covering, internal

22

24

25

26

27

Deck top covering, internal (linoleum, tiles, parquet etc.)

Floor plates (loose), platforms, steps, ladders, etc. in accommodation

Floor plates (loose), platforms, steps, ladders, etc. in Pontoon

Ladders, platforms, railings, etc. in columns

Floors and platforms in machinery spaces

Group 53 – External deck covering, steps, ladders etc., fore-and-aft gangway

5

EQUIPMENT FOR CREW

EXTERNAL DECK COVERING, STEPS, LADDERS, ETC, FORE-AND-AFT GANGWAY

53

5

5

5

5

5

5

31

Deck covering, external

32

33

34

35

39

Wood deck, external

Handrails, railings and rail gates

Ladders, steps, platforms, gratings with equipment, external

Fore-and-aft gangways with equipment

Ladders and platforms external on columns

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

20 of 95

Group 54 – Furniture, inventory and entertainment equipment

5

EQUIPMENT FOR CREW

FURNITURE, INVENTORY AND ENTERTAINMENT EQUIPMENT

54

5

5

5

5

5

5

5

41

Furniture for crew, standard furniture

Office equip. & spec. furniture in wheelhouse, chartroom, etc.

Bedclothes and mattresses

42

43

44

45

46

47

Curtains with equipment, carpets, etc.

Decorations, figureheads

Hobby, sports and entertainment equipment, etc.

Inventory, smaller items

Group 55 – Galley & pantry equip., arrangement for provision, ironing/ drying

equipment

5

EQUIPMENT FOR CREW

GALLEY & PANTRY EQUIPMENT , ARRANGEMENT FOR PROVISIONS,

IRONING/DRYING EQUIPMENT

55

5

5

5

5

5

5

5

51

Galley machinery

52

53

54

55

56

58

Equipment/outfitting in galley and pantry

Cafeteria plant with equipment

Freezing and refrigerating systems for provisions

Insulation, lining & battening in prov. rooms, insul. doors, etc.

Bulkheads, bins, shelves, racks etc. in provision rooms

Ironing and drying equipment, laundry

Group 56 – Lifting and transport equipment for crew and provisions

5

EQUIPMENT FOR CREW

LIFTING AND TRANSPORT EQUIPMENT FOR CREW AND PROVISIONS

56

5

5

5

61

Personnel lifts, escalators

64

66

Shore gangway with davits, etc. landing booms

Helicopter platform with equipment

Group 57 – Ventilation, air-conditioning and heating systems

5

EQUIPMENT FOR CREW

VENTILATION, AIR-CONDITIONING AND HEATING SYSTEMS

57

5

5

5

5

5

71

Ventilation/air-conditioning systems for accommodation, etc.

Ventilation/air-conditioning systems for provision rooms, etc.

Ventilation/air-conditioning systems for control rooms, etc.

Ventilation/air-conditioning systems for machinery spaces

Ventilation/air-conditioning systems for mud rooms, shale shaker room, etc.

72

73

74

75

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

21 of 95

5

EQUIPMENT FOR CREW

VENTILATION, AIR-CONDITIONING AND HEATING SYSTEMS

57

5

5

5

76

Ventilation/air-conditioning systems for Co2 room, store-rooms, etc.

Central heating system with radiators & fan heaters

77

78

Ventilation/air-conditioning systems in rooms for drilling machinery

Group 58 – Sanitary systems with discharges

5

EQUIPMENT FOR CREW

SANITARY SYSTEMS WITH DISCHARGES

58

5

5

5

5

81

Sanitary supply systems

82

83

84

Sanitary discharge systems, drainage systems

Bathtubs, bidets, shower cabinets, WC, washbasins, etc.

Drinking water systems and coolers

Main Group 6 - Machinery Main Components

Group 62 – Other types of propulsion machinery

6

MACHINERY MAIN COMPONENTS

OTHER TYPES OF PROPULSION MACHINERY

Electric generator - motor plants etc.

62

6

25

Group 63 – Transmission and foils

6

MACHINERY MAIN COMPONENTS

TRANSMISSIONS AND FOILS

63

6

6

6

31

Fixed propeller plant

35

37

Azimuth Thrusters

Main reduction gears with thrust bearings, couplings

Group 64 – Boilers, steam and gas generators

6

MACHINERY MAIN COMPONENTS

BOILERS, STEAM AND GAS GENERATORS

Boilers, steam and gas generators

64

6

44

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

22 of 95

Group 65 – Motor aggregates for main electric power production

6

MACHINERY MAIN COMPONENTS

MOTOR AGGREGATES FOR MAIN ELECTRIC POWER PRODUCTION

Main Diesel Generator Engines

6

5

6

51

Group 66 – Other aggregates & generators for main & emergency electrical power

production

6

MACHINERY MAIN COMPONENTS

OTHER AGGREGATES & GENERATORS FOR MAIN & EMERGENCY ELECTRICAL POWER

PRODUCTION

66

6

65

Emergency Diesel Generator Engines

Main Group 7 - Systems for Machinery Main Components

Group 70 – Fuel systems

7

SYSTEMS FOR MACHINERY MAIN COMPONENTS

FUEL SYSTEMS

70

7

7

7

7

01

Fuel oil transfer and drain systems

Fuel oil purification plants

Fuel oil supply system

02

03

04

Heating coils in fuel tanks

Group 71 – Lube oil systems

7

SYSTEMS FOR MACHINERY MAIN COMPONENTS

LUBE OIL SYSTEMS

71

7

7

7

7

7

11

Lube oil transfer and drain systems

12

13

14

15

Lube oil purification plants

Lube oil system for propulsion machinery and transmissions

Lube oil systems for motor/turbo aggregates, etc.

Common lube oil system for other machinery

Group 72 – Cooling systems

7

SYSTEMS FOR MACHINERY MAIN COMPONENTS

COOLING SYSTEM

72

7

7

21

22

Sea water cooling system

Fresh water and other cooling Systems

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

23 of 95

Group 73 – Compressed air systems

7

SYSTEMS FOR MACHINERY MAIN COMPONENTS

COMPRESSED AIR SYSTEMS

73

7

7

7

31

Starting air systems (high pressure)

General purpose air system (low pressure)

Instrument air supply system

33

34

Group 74 – Exhaust systems and air intakes

7

SYSTEMS FOR MACHINERY MAIN COMPONENTS

EXHAUST SYSTEM AND AIR INTAKES

74

7

7

7

41

Fresh air intakes (not ventilation)

44

45

Exhaust gas systems for diesel generator engines, etc.

Exhaust gas system for boilers, etc.

Group 76 – Distilled and make-up water systems

7

Systems SYSTEMS FOR MACHINERY MAIN COMPONENTS

DISTILLED AND MAKE-UP WATER SYSTEMS

Distilled and make-up water systems

76

7

61

Group 79 – Automation systems for machinery

7

SYSTEMS FOR MACHINERY MAIN COMPONENTS

AUTOMATION SYSTEMS FOR MACHINERY

79

7

7

7

7

7

7

7

91

Manoeuvre consoles, main consoles

92

93

94

95

96

97

Common automation equipment (engine room alarm, etc.)

Automation equipment for propulsion machinery and transmissions

Automation equipment for boilers, etc.

Automation equipment for diesel generator engines etc.

Power management system

Automation equipment for other machinery components

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

24 of 95

Main Group 8 - Platform Common Systems

Group 80 – Ballast and bilge systems, gutter pipes outside accommodation

8

PLATFORM COMMON SYSTEMS

BALLAST AND BILGE SYSTEMS, GUTTER PIPES OUTSIDE ACCOMMODATION

80

8

8

8

8

01

Ballast Systems, solid ballast

Heating coils in ballast tanks

Bilge systems

02

03

04

Gutter pipes outside accommodation

Group 81 – Fire & lifeboat alarm, fire- fighting & wash down systems

8

PLATFORM COMMON SYSTEMS

FIRE & LIFEBOAT ALARM, FIRE-FIGHTING & WASH DOWN SYSTEMS

81

8

8

8

8

8

8

8

11

Fire detection, fire and lifeboat alarm systems

Emergency shutdown system (ESD)

12

13

14

15

16

17

Fire/wash down systems, emergency fire pumps, general service pumps

Fire-fighting systems for external fires

Fixed fire-fighting system with CO2

Fire-fighting systems with foam

Fixed fire-fighting system with inergen / water mist system or similar

Group 82 – Air and sounding systems from tanks to deck

8

PLATFORM COMMON SYSTEMS

AIR AND SOUNDING SYSTEMS FROM TANKS TO DECK

Air and sounding systems from tanks to deck

8

2

8

21

Group 83 – Special common hydraulic oil systems

8

PLATFORM COMMON SYSTEMS

SPECIAL COMMON HYDRAULIC OIL SYSTEMS

Special common hydraulic oil systems

8

3

8

31

Group 84 – Central heat transfer systems with chemical liquids

8

PLATFORM COMMON SYSTEMS

CENTRAL HEAT TRANSFER SYSTEMS WITH CHEMICAL LIQUIDS

84

8

8

41

42

Heating oil treatment system

Heating oil distribution systems for engine room

Doc. ID (rev.)

T-MA-10672(5)

Document Title

Page

Appendix A - Engineering Numbering System Manual

25 of 95

Group 85 – Automation common systems

8

PLATFORM COMMON SYSTEMS

AUTOMATION COMMON SYSTEMS

85

8

8

8

8

51

Integrated automation system (IAS)

52

53

54

Automation system for watertight integrity

Automation system for ventilation-, air-conditioning- and heating systems

Automation system for fire-fighting- & wash down systems

Group 86 – Electrical power supply

8

PLATFORM COMMON SYSTEMS

ELECTRICAL POWER SUPPLY

86

8

8

8

8

8

61

Generators/alternators

Transformers

65

66

67

68

Batteries and chargers

Converters and rectifiers

Shore supply

Group 87 – Electrical distribution common systems

8

PLATFORM COMMON SYSTEMS

ELECTRICAL DISTRIBUTION COMMON SYSTEMS

87

8

8

8

8

8

71

Main switchboard

72

73

74

75

Emergency switchboard

Group starters

Local starters

Distribution panels and boards

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

26 of 95

A.4 NSFI Detail description and guide

Main Group 2 - Hull and Structure

Group 26 – Hull outfitting

2

HULL AND STRUCTURE

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

HULL OUT FITTING

26

2

62 Drain-, bottom plugs, sea chests, manholes,

watertight covers and hatches

Doc: NNN-100-A-AA-NNN (.NN)

Doc: NNN-101-A-AA-NNN (.NN)

Doc: NNN-200-A-AA-NNN (.NN)

Doc: NNN-260-A-AA-NNN (.NN)

Doc: NNN-262-A-AA-NNN (.NN)

NSFI description: Surveyor’s hatches, waterproof covers etc.

with coaming, packings, bolts and fittings. Complete sea chest

and bilge wells. Bottom plugs for draining of hull tanks.

Types of asset:

Function and type code according to Appendix B:

-

Manholes

Tag: 262-OT-NNN

OT (Manholes)

2

HULL AND STRUCTURE

HULL OUT FITTING

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

26

2

64 Boat bumpers, fender bars and wear bars

Doc: NNN-100-A-AA-NNN (.NN)

Doc: NNN-101-A-AA-NNN (.NN)

Doc: NNN-200-A-AA-NNN (.NN)

Doc: NNN-260-A-AA-NNN (.NN)

Doc: NNN-264-A-AA-NNN (.NN)

NSFI description: Permanent hull outfitting, external, such as

boat bumpers, bilge keels, fender bars and wear bars, ice

fins/knives and grating. Spray plates.

Types of asset:

Function and type code according to Appendix B

-

Fenders (boat fenders/bumpers)

Tag: 264-YO-NNN

YO (Fenders)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

27 of 95

Main Group 3 - Drilling Equipment and Systems

Group 30 – Derrick with Components

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

The group includes complete derrick structure and all primary

hoisting equipment, both stationary and travelling parts down to

Kelly.

DERRICK WITH COMPONENTS

30

3

01 Derrick structure

Doc: NNN-100-A-AA-NNN (.NN)

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-301-A-AA-NNN (.NN)

NSFI description: Standard derrick including all equipment not

being part of the hoisting system. Such as derrick steel structure,

platforms and boards, ladders, guide rails, rescue line,

counterweights, line stabilizers, light fixture, etc.

Function and type code according to Appendix B:

Types of asset:

-

-

-

Derrick Hoisting Rig System

Tag: 301-BA-NNN

Tag: 301-BB-NNN

Tag: 301-LS-NNN

BA (Derrick Hoisting Rig System)

Cylinder Based Hoisting Rig System

Yoke Sheaves

BB (Cylinder Based Hoisting Rig System)

LS (Yoke Sheaves)

3

3

02 Crown block with drill string motion

compensator

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-302-A-AA-NNN (.NN)

NSFI description: Complete crown block assembly including

crown block, misc. sheaves at water table and gin pole. Also

included possible crown block mounted heave compensator.

Function and type code according to Appendix B

Types of asset:

-

Crown Block

Tag: 302-LC-NNN

LC (Crown Block)

03 Travelling equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-303-A-AA-NNN (.NN)

NSFI description: Travelling equipment in derrick, including

travelling block, hook and guide dolly.

Function and type code according to Appendix B

LB (Travelling Block)

Types of asset:

-

-

-

-

-

Travelling block

Guide Track and Dolly

Travelling Yoke

Rotary Swivel

Tag: 303-LB-NNN

Tag: 303-LG-NNN

Tag: 303-LY-NNN

Tag: 303-LN-NNN

Tag: 303-JD-NNN

LG (Guide Track and Dolly)

LY (Travelling Yoke)

LN (Rotary Swivel)

Drill Line Drum

JD (Drill Line Drum)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

28 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

DERRICK WITH COMPONENTS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

The group includes complete derrick structure and all primary

hoisting equipment, both stationary and travelling parts down to

30

Kelly.

3

04 Drill string heave compensator

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-304-A-AA-NNN (.NN)

NSFI description: Includes travelling block mounted

compensator unit w/ main components, hydraulic system and

control console.

Types of asset:

Function and type code according to Appendix B

YD (Drill String Compensators)

-

-

-

-

-

-

Compensators (AHC, CMC etc.)

HPU for Compensators

APV for Compensators

Control Console

Tag: 304-YD-NNN

Tag: 304-CT-NNN

Tag: 304-VB-NNN

Tag: 304-YC-NNN

Tag: 304-VN-NNN

Tag: 304-YF-NNN

CT (Hydraulic Power Units (HPU))

VB (Accumulators)

YC (Drill String Control System)

Nitrogen Pressure Vessels

Rocker Arms

VN (Nitrogen Units)

YF (Dual Rocker Arm System)

3

05 Miscellaneous derrick systems

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-305-A-AA-NNN (.NN)

NSFI description: Derrick mounted systems and components

not mentioned in the previous sub-groups. Including safety

elevator for derrick personnel, piping etc.

Function and type code according to Appendix B

MC (Elevators and Lifts)

Types of asset:

-

-

Safety elevator (for personnel)

Access baskets

Tag: 305-MC-NNN

Tag: 305-LQ-NNN

LQ (Service access baskets)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

29 of 95

Group 31 – Drill floor equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes drill floor mounted equipment and systems

and associated control devices.

DRILL FLOOR EQUIPMENT AND SYSTEMS

31

3

11 Drilling Control

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-310-A-AA-NNN (.NN)

Doc: NNN-311-A-AA-NNN (.NN)

NSFI description: All instruments and control devices for

surveying the drilling operation. Included are drilling recorder,

driller console w/ instruments and operating devices, etc.

Function and type code according to Appendix B

Types of asset:

-

Drilling Panels/Drillers Console

Tag: 311-IE-NNN

IE (Desk/Consoles, operator stations, work stat., PC, VDU))

3

12 Drawworks & machinery

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-310-A-AA-NNN (.NN)

Doc: NNN-312-A-AA-NNN (.NN)

NSFI description: Drawwork with all accessory equipment such

as gear unit, main and auxiliary brakes, crown-o-matic, sandline

drum and cathead(s).

Types of asset:

Function and type code according to Appendix B

-

-

Drawwork Package

Eddy Current Brake

Tag: 312-BG-NNN

Tag: 312-BH-NNN

BG (Draw works Package)

BH (Drawworks Brake Packages)

3

3

13 Rotary table, top drive and associated

equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-310-A-AA-NNN (.NN)

Doc: NNN-313-A-AA-NNN (.NN)

NSFI description: Machinery for rotating the drill string, such

as: Rotary table w/accessories, top drive, master bushing

w/bowls, Kelly drive bushing, skid, etc.

Function and type code according to Appendix B

BT (Rotary Table)

Types of asset:

-

-

Rotary Table Packages

Tag: 313-BT-NNN

Tag: 313-BJ-NNN

DDM-1000-HY-RR/DDM-650-HY-RR

BJ (Top Drive Package (PDT/DDM/MDDM))

14 Tensioning systems

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-310-A-AA-NNN (.NN)

Doc: NNN-314-A-AA-NNN (.NN)

NSFI description: Systems for tensioning of marine riser,

guidelines, pod lines, etc., including tensioners, sheaves and

control equipment.

Types of asset:

Function and type code according to Appendix B

YA (Wire Type Riser Tensioner)

-

-

-

Riser Tensioners

Tag: 314-YA-NNN

Tag: 314-YB-NNN

Tag: 314-YI-NNN

Guide- and Pod Line Tensioners

Idler Sheaves

YB (Guide- and Pod line Tensioners)

YI (Idler Sheaves)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

30 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

DRILL FLOOR EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes drill floor mounted equipment and systems

and associated control devices.

31

3

15 H.P. air system for tensioners and

compensators

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-310-A-AA-NNN (.NN)

Doc: NNN-315-A-AA-NNN (.NN)

NSFI description: Separate system for supply of compressed

air to equipment for tensioning and heave compensation, such as

riser- and guideline tensioners, drillstring heave compensator,

etc.

Types of asset:

Function and type code according to Appendix B

See K (Compressors, Blowers and Expanders).

VK (Dryers)

-

-

-

-

HP Air Compressor

Dryers

Tag: 315-KA-NNN

Tag: 315-VK-NNN

Tag: 315-VN-NNN

Tag: 315-XY-NNN

APV for Tensioners

Nitrogen Generator Packages

VN (Nitrogen Units)

XY (Nitrogen Generator Packages)

3

3

16 Wire lines

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-310-A-AA-NNN (.NN)

Doc: NNN-316-A-AA-NNN (.NN)

NSFI description: All wire lines associated to the drilling

operation, such as: Drilling- and sandline, storage reel, wire lines

for tensioners, etc.

Function and type code according to Appendix B

JP (Wire Reels)

Types of asset:

-

Wire Reels

Tag: 316-JP-NNN

17 Miscellaneous drillfloor equipment and

systems

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-310-A-AA-NNN (.NN)

Doc: NNN-317-A-AA-NNN (.NN)

NSFI description: Miscellaneous equipment and systems on

drill floor not mentioned in the previous sub-groups, such as:

Mouse- and rathole assembly, deadline anchor w/loadcell, etc.

Function and type code according to Appendix B

LA (Deadline Anchor)

Types of asset:

-

-

-

Deadline Anchor

Tag: 317-LA-NNN

Tag: 317-LH-NNN

Tag: 317-LU-NNN

Mouse- and rathole assembly

Utility Winches

LH (Mouseholes, Ratholes, Conductor holes)

LU (Utility Winches)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

31 of 95

Group 32 – Bulk and Mud Systems incl. Cement, Brine & Base

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes complete system for drilling and mud

BULK AND MUD SYSTEMS INCL. CEMENT,

BRINE & BASE

21 Bulk storage and transfer systems

storage, circulation and solids control. Also including system for

storage and transport of dry bulk material and drill water.

NSFI description: Tank for storage of dry cement and mud

additives. Separate transfer system with bunkering hoses,

ejectors and pumps as well as piping and pipe fittings for

intermediate tank transport and rig unloading purposes.

Function and type code according to Appendix B

32

3

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-320-A-AA-NNN (.NN)

Doc: NNN-321-A-AA-NNN (.NN)

Types of asset:

-

-

-

-

Dust Cyclones

Dust Collector Tanks

Surge Tanks

Tag: 321-CE-NNN

Tag: 321-VL-NNN

Tag: 321-VL-NNN

Tag: 321-CQ-NNN

CE (Cyclones)

VL (Receiver and Surge Drums, Expansion-Head Tanks)

VL (Receiver and Surge Drums, Expansion-Head Tanks)

CQ (Eductors and Ejectors)

Ejectors

3

3

22 Bulk air system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-320-A-AA-NNN (.NN)

Doc: NNN-322-A-AA-NNN (.NN)

NSFI description: Pneumatic system for bulk-handling.

Including power units, hoses, valves etc.

Types of asset:

Function and type code according to Appendix B

K (Compressors, Blowers and Expanders).

See Appendix B, Section K for type of compressors.

-

Bulk Air Compressor

Tag: 322-KA-NNN

23 Mud additive system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-320-A-AA-NNN (.NN)

Doc: NNN-323-A-AA-NNN (.NN)

NSFI description: System for metering, weighing and adding

chemicals to the drilling fluid. Including misc. instruments,

sensors, remotely operated valve, control system etc.

Function and type code according to Appendix B

Types of asset:

-

Big Bag Station

Tag: 323-BV-NNN

BV (Big Bag Unit)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

32 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes complete system for drilling and mud

BULK AND MUD SYSTEMS INCL. CEMENT,

BRINE & BASE

24 Mud mixing and storage

storage, circulation and solids control. Also including system for

storage and transport of dry bulk material and drill water.

NSFI description: Complete system for mixing dry mud, water

and additives as well as for internal recirculation of mud in pits.

Included are mud mixing pumps, mud hoppers, mud pits etc.

with associated piping and fittings, recirculation lines and locally

mounted control equipment. Also included in this are mud guns

and agitators for mud conditioning.

32

3

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-320-A-AA-NNN (.NN)

Doc: NNN-324-A-AA-NNN (.NN)

Types of asset:

Function and type code according to Appendix B

-

-

-

-

-

-

-

-

-

Mud Mix Hoppers

Tag: 324-BY-NNN

Tag: 324-BY-NNN

Tag: 324-BY-NNN

Tag: 324-CJ-NNN

Tag: 324-CX-NNN

Tag: 324-BZ-NNN

Tag: 324-TF-NNN

Tag: 324-TA-NNN

Tag: 324-TB-NNN

BY (Hoppers/Mud Guns/Blenders)

High Rate Mixers

BY (Hoppers/Mud Guns/Blenders)

HP Mud Shear Guns

BY (Hoppers/Mud Guns/Blenders)

Agitators for Mud Mixing and Storage

Tank Washers

CJ (Mixers/Agitators)

CX (Other Miscellaneous Mechanical Equipment)

BZ (Other Drilling Equipment Packages)

Mud Mixing and Storage Packages

Mud Pits

TF (Mud Pits)

Storage Tanks - Cylindrical

Storage Tanks - Rectangular

TA (Storage Tanks - Cylindrical)

TB (Storage Tanks - Rectangular)

3

25 Mud supply

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-320-A-AA-NNN (.NN)

Doc: NNN-325-A-AA-NNN (.NN)

NSFI description: System for supply of conditioned mud from

mud pits to swivel. Included are all pumps concerned with mud

supply and processing, pulsation dampers, pressure relief valves,

standpipe manifold, standpipe, rotary hoses and swivel.

Function and type code according to Appendix B

Types of asset:

-

H.P. Mud Pump Package

Tag: 325-BK-NNN

Tag: 325-BS-NNN

BK (Mud Pump Package)

-

-

Standpipe Manifold

BS (Choke/Production/Injection & Test Manifold)

Mud Charge Pumps (Centrifugal)

PA (Centrifugal Pumps), P (Pumps)

Comment: See Section 6.1 (Main document) and Appendix B.1

Tag: 324-PA-NNN

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

33 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes complete system for drilling and mud

BULK AND MUD SYSTEMS INCL. CEMENT,

BRINE & BASE

26 Mud solids control

storage, circulation and solids control. Also including system for

storage and transport of dry bulk material and drill water.

NSFI description: System for mud solids control. Included are

mud flowline w/flowmeter, shale shaker, degasser, desander,

desilter, possible separators, mud treatment tanks incl. trip tank,

hoses, piping and valves. Also included are special systems for

solids control of oil based mud and drain system for separated

mud/cuttings.

32

3

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-320-A-AA-NNN (.NN)

Doc: NNN-326-A-AA-NNN (.NN)

Types of asset:

Function and type code according to Appendix B

-

-

-

-

-

-

-

Shale Shaker Units

Centrifuges

Tag: 326-CF-NNN

Tag: 326-CC-NNN

Tag: 326-CJ-NNN

Tag: 326-CF-NNN

Tag: 326-NC-NNN

Tag: 326-PA-NNN

Tag: 326-PG-NNN

CF (Mechanical Separators / Shakers)

CC (Centrifuges)

Mixers/Agitators

Degasser

CJ (Mixers/Agitators)

CF (Mechanical Separators / Shakers)

Cuttings Conveyor

Centrifugal Pumps

Screw Pumps

NC (Screw Conveyors/Feeders)

PA (Centrifugal Pumps)

PG (Screw Pumps)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

34 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes complete system for drilling and mud

storage, circulation and solids control. Also including system for

storage and transport of dry bulk material and drill water.

NSFI description: Complete drill water system, incorporating

equipment for drill water bunkering, storage, transfer and

consumption.

BULK AND MUD SYSTEMS INCL. CEMENT,

BRINE & BASE

27 Drill water system

32

3

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-320-A-AA-NNN (.NN)

Doc: NNN-327-A-AA-NNN (.NN)

Types of asset:

Function and type code according to Appendix B

PA (Centrifugal Pumps)

-

-

Centrifugal Pumps

Tag: 327-PA-NNN

Tag: 327-TB-NNN

Drill Water Storage Tanks - Hull

TB (Storage Tanks - Rectangular)

3

28 H.P. hoses, chicksans, piping and valves

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-320-A-AA-NNN (.NN)

Doc: NNN-328-A-AA-NNN (.NN)

NSFI description: Miscellaneous bulk/mud system components

not belonging to a specific sub-group.

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

35 of 95

Group 33 – Well control equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes equipment for control of well pressure and

flow, such as blowout preventer (BOP) stacks with necessary

control equipment, diverter, riser and choke & kill system.

NSFI description: All stacks for blowout prevention purposes

including BOP guide frame assembly, wellhead connector,

ball/flexible joints, stack mounted kill and choke fail safe valve

and connection clamps.

WELL CONTROL EQUIPMENT AND SYSTEMS

33

3

31 Blowout preventer stack

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-330-A-AA-NNN (.NN)

Doc: NNN-331-A-AA-NNN (.NN)

Types of asset:

Function and type code according to Appendix B

-

-

-

-

Subsea BOP Assembly

BOP Stack

Tag: 331-BD-NNN

Tag: 331-WA-NNN

Tag: 331-WB-NNN

Tag: 331-WC-NNN

BD (Subsea BOP Assembly)

WA (BOP Stack)

Lower Riser Marine Preventer (LRMP)

Wellhead Connector

WB (Lower Riser Marine Preventer (LRMP))

WC (Wellhead Connector)

3

32 Blowout preventer control system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-330-A-AA-NNN (.NN)

Doc: NNN-332-A-AA-NNN (.NN)

NSFI description: Equipment for hydraulic power generation

and signal control on surface, power and signal transmission to

BOP stack, and sub-sea control equipment for operation of stack

functions. Included are hydraulic power unit with reservoir and

accumulator banks, interruptible electric power supply, surface

control unit with panels, control/power hoses with reels, acoustic

transducers, sub-sea control pods/control units, stack mounted

accumulators and shuttle valves. Also included are necessary

pressure and function testing equipment.

Types of asset:

Function and type code according to Appendix B

-

-

-

-

BOP Control System

Tag: 332-BC-NNN

Tag: 332-WD-NNN

Tag: 332-WE-NNN

Tag: 332-WY-NNN

BC (BOP Control System)

Surface BOP Control System

Surface Acoustic BOP Control System

Subsea BOP Control POD System

WD (Surface BOP Control System)

WE (Surface Acoustic BOP Control System)

WY (Subsea BOP Control POD System)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

36 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

WELL CONTROL EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes equipment for control of well pressure and

flow, such as blowout preventer (BOP) stacks with necessary

control equipment, diverter, riser and choke & kill system.

NSFI description: System for connection riser directly in to

wellhead when drilling in shallow gas sands. Including 30”

3

33 Shallow gas stack with control system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-330-A-AA-NNN (.NN)

Doc: NNN-333-A-AA-NNN (.NN)

hydraulic latch with control system.

Types of asset:

Function and type code according to Appendix B

-

-

-

Shallow Gas Stack

Tag: 333-WI-NNN

Tag: 333-WJ-NNN

Tag: 333-JQ-NNN

WI (Shallow Gas Stack)

Shallow Gas Control System

Quick Latch Unions

WJ (Shallow Gas Control System)

JQ (Quick Latch Unions)

3

3

34 Diverter with control systems

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-330-A-AA-NNN (.NN)

Doc: NNN-334-A-AA-NNN (.NN)

NSFI description: System for diverting possible gas kicks from

shallow pockets away from the drilling rig. Includes diverter

assembly, ventline w/vales and diverter control system w/panels.

Function and type code according to Appendix B

Types of asset:

-

Diverter Package

Tag: 334-BN-NNN

BN (Diverter Package)

35 Riser systems incl. choke, kill and booster

lines

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-330-A-AA-NNN (.NN)

Doc: NNN-335-A-AA-NNN (.NN)

NSFI description: Complete riser with riser sections, Choke &

Kill lines/hoses, booster lines, slip joint, spider assembly and

handling tools.

Types of asset:

Function and type code according to Appendix B

-

-

-

-

-

-

-

-

-

Drilling Riser Packages

Tag: 335-BR-NNN

Tag: 335-QJ-NNN

Tag: 335-QP-NNN

Tag: 335-WR-NNN

Tag: 335-WQ-NNN

Tag: 335-WS-NNN

Tag: 335-WU-NNN

Tag: 335-JS-NNN

Tag: 335-JG-NNN

BR (Drilling Riser Package)

Riser Test Tool

QJ (Riser Test Tool)

Workover Riser System Equipment

Support ring (Riser Tensioner ring)

Slip joint / Telescopic joint

Support Ring Handling Tool

Riser Handling Tool (Riser Running Tool)

Riser Spider

QP (Workover Riser System Equipment)

WR (Support ring (Riser Tensioner ring)

WQ (Slip joint / Telescopic joint)

WS (Support Ring Handling Tool)

WU (Support Ring Handling Tool)

JS (Riser Spider)

Gimbal for Riser Spider

JG (Gimbal for Riser Spider)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

37 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

WELL CONTROL EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes equipment for control of well pressure and

flow, such as blowout preventer (BOP) stacks with necessary

control equipment, diverter, riser and choke & kill system.

NSFI description: A standby mud flowpath, primarily for

circulating a kick. Included are choke and kill manifold with

chokes, piping and goose necks, mud/gas separator, valves and

necessary control and operation equipment.

Function and type code according to Appendix B

3

36 Choke and kill systems incl. mud/gas

separator

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-330-A-AA-NNN (.NN)

Doc: NNN-336-A-AA-NNN (.NN)

Types of asset:

-

-

-

Poor Boy Degasser (Atmospheric)

Mud Gas Separator

Tag: 336-WV-NNN

Tag: 336-WW-NNN

Tag: 336-BS-NNN

WV (Poor Boy Degasser (Atmospheric))

WW (Mud Gas Separator)

Choke & Kill manifold

BS (Choke & Kill manifold)

3

37 Miscellaneous well control related

equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-330-A-AA-NNN (.NN)

Doc: NNN-337-A-AA-NNN (.NN)

NSFI description: Miscellaneous equipment primarily for well

control purposes, such as: Inside BOP, drill string safety valves,

BOP test tool, control valves, BOP pressure test pump system

etc.

Types of asset:

Function and type code according to Appendix B

-

BOP Control System Test Equipment

Tag: 337-WG-NNN

WG (BOP Control System Test Equipment)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

38 of 95

Group 34 – Pipe handling equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes lifting and transport equipment for vertical

and horizontal displacement of drill pipes and tubulars.

PIPE HANDLING EQUIPMENT AND SYSTEMS

34

3

41 Derrick mounted vertical pipe handling

system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-340-A-AA-NNN (.NN)

Doc: NNN-341-A-AA-NNN (.NN)

NSFI description: Derrick installed system for

handling/positioning of vertical pipes. Including racking arms,

fingerboard, drill floor manipulators, travelling block retractor,

etc.

Types of asset:

Function and type code according to Appendix B

JB (Mud Buckets)

-

-

-

-

-

-

Mud Buckets

Tag: 341-JB-NNN

Tag: 341-JR-NNN

Tag: 341-LF-NNN

Tag: 341-LM-NNN

Tag: 341-LR-NNN

Tag: 341-LV-NNN

Hydraulic Roughneck

Fingerboards

JR (Hydraulic Roughneck)

LF (Fingerboards)

Manipulator Arms

LM (Manipulator Arms)

Racking Equipment

Horizontal to Vertical Arms (HTV)

LR (Racking Equipment)

LV (Horizontal to Vertical Arms (HTV))

3

42 Drill floor tubular handling equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-340-A-AA-NNN (.NN)

Doc: NNN-342-A-AA-NNN (.NN)

NSFI description: Equipment for make-up/break of tubulars on

drill floor. Including spinners, iron Roughnecks, easy-torque,

power tongs, etc.

Types of asset:

Function and type code according to Appendix B

-

-

-

-

Hydraulic Roughneck

Tag: 342-JR-NNN

Tag: 342-JH-NNN

Tag: 342-JT-NNN

Tag: 342-JW-NNN

JR (Hydraulic Roughneck)

Hydraulic Catheads

JH (Hydraulic Catheads)

Hydraulic Torque Wrench / Power Tongs

Drill pipe thread washer/doper

JT (Hydraulic Torque Wrench / Power Tongs)

JW (Drill pipe thread washer/doper)

3

43 Power packs for pipe/tubular handling

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-340-A-AA-NNN (.NN)

Doc: NNN-343-A-AA-NNN (.NN)

NSFI description: Complete hydraulic/pneumatic power packs

for hoisting/positioning during pipe/tubular handling. Including

power units, hoses, valves, etc.

Types of asset:

Function and type code according to Appendix B

-

Hydraulic Power Units (HPU)

Tag: 343-CT-NNN

CT (Hydraulic Power Units (HPU))

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

39 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

PIPE HANDLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes lifting and transport equipment for vertical

and horizontal displacement of drill pipes and tubulars.

34

3

44 Slips and spiders

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-340-A-AA-NNN (.NN)

Doc: NNN-344-A-AA-NNN (.NN)

NSFI description: Including manual and power slips, spiders

and drill collar safety clamp.

Types of asset:

Function and type code according to Appendix B

-

Power Slips

Tag: 344-JJ-NNN

JJ (Power Slips)

3

45 Elevators

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-340-A-AA-NNN (.NN)

Doc: NNN-345-A-AA-NNN (.NN)

NSFI description: Elevators for drill pipes, drill collars and

casing, elevator link.

Types of asset:

Function and type code according to Appendix B

-

BX Elevators

Tag: 345-LI-NNN

LI (Elevators)

3

3

46 Manual tongs

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-340-A-AA-NNN (.NN)

Doc: NNN-346-A-AA-NNN (.NN)

NSFI description: Manual tongs/wrenches for drill pipe. Also

including torque indicators.

Types of asset:

Function and type code according to Appendix B

JM (Manual Tongs)

-

Manual Tongs

Tag: 346-JM-NNN

47 Miscellaneous equipment for pipe/tubular

handling

Doc: NNN-347-AA-NNN

Drw: NNN-347-D-AA-NNN.(NN)

NSFI description: Miscellaneous equipment for pipe/tubular

handling that has not been referred to a specific sub-group, such

as: Stabbing board etc.

Types of asset:

Function and type code according to Appendix B

LK (Casing Stabbing Board)

-

-

Casing Stabbing Board

Tag: 347-LK-NNN

Tag: 347-LT-NNN

Tubular Feeding Machines (TFM)

LT (Tubular Feeding Machines (TFM))

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

40 of 95

Group 35 – Drill string and downhole equipment

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

NSFI Description / Tag description / Comments

This group includes the complete drill string (drill pipes, drill

collars, Kelly) and downhole tools and equipment.

DRILL STRING AND DOWNHOLE EQUIPMENT [Main document]

35

3

51 Drill pipes

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-350-A-AA-NNN (.NN)

Doc: NNN-351-A-AA-NNN (.NN)

NSFI description: Drill pipes with tool joints and pup joints.

3

52 Drill collars

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-350-A-AA-NNN (.NN)

Doc: NNN-352-A-AA-NNN (.NN)

NSFI description: All drill collars, either round, square,

triangular or spiral groove shaped. Also including non-magnetic

and stub drill collars.

3

3

3

3

3

53 Substitutes (tubular)

54 Stabilizers

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-350-A-AA-NNN (.NN)

Doc: NNN-353-A-AA-NNN (.NN)

NSFI description: Cross-over subs, bit subs, bumper subs,

saver subs, baskets, lift plugs for lifting heavy tubular goods, etc.

lift subs.

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-350-A-AA-NNN (.NN)

Doc: NNN-354-A-AA-NNN (.NN)

NSFI description: Devices for stiffening/stabilizing the drill

string, such as: Sleeve-type stabilizers, integral blade stabilizers,

rubber sleeve stabilizers, etc.

55 Downhole drilling tools

56 Fishing tools

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-350-A-AA-NNN (.NN)

Doc: NNN-355-A-AA-NNN (.NN)

NSFI description: A variety of tools to perform specialized

bottom-end operations during drilling. Examples are hole

openers, under-reamers, bits, core-barrels, etc.

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-350-A-AA-NNN (.NN)

Doc: NNN-356-A-AA-NNN (.NN)

NSFI description: Tools designed for the specific purpose of

recovering lost equipment form the well, such as overshots, jars,

junkshots, junk baskets, mills, cutters, etc.

57 Misc. drill string and downhole equipment

(incl. kelly)

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-350-A-AA-NNN (.NN)

Doc: NNN-357-A-AA-NNN (.NN)

NSFI description: Miscellaneous drill string and downhole

equipment not belonging to a specific sub-group. In addition to

Kelly also including deviation drilling tools/equipment, etc.

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

41 of 95

Group 36 – Material handling equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes lifting and transport equipment, primarily for

drilling equipment.

MATERIAL HANDLING EQUIPMENT AND

SYSTEMS

36

3

61 Deck cranes

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-360-A-AA-NNN (.NN)

Doc: NNN-361-A-AA-NNN (.NN)

NSFI description: Deck cranes mainly for loading/unloading

and stowage of drilling equipment, stores, etc. Included are

complete cranes with pedestal, revolving structure, boom, main

and whip hoists, swing system, operators cabin, instruments and

power units (el.motor or diesel engine). Also included are luffing

and sleewing winches, hooks, limit switches, etc.

Function and type code according to Appendix B

Types of asset:

-

Pedestal Cranes

Tag: 361-MA-NNN

MA (Pedestal Cranes)

3

62 Overhead cranes

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-360-A-AA-NNN (.NN)

Doc: NNN-362-A-AA-NNN (.NN)

NSFI description: Travelling cranes on rails. Normally for

carrying pipe, casing and riser sections from pipe rack to skidway

and vice versa.

Types of asset:

Comment: Function and type code according to Appendix B

-

-

Overhead Cranes

Gantry Cranes

Tag: 362-MF-NNN

Tag: 362-MV-NNN

MF (Overhead Cranes)

MV (Gantry and Portal Cranes)

Examples of asset:

Comment: According to Building Specification

Tag description: Crane, Gantry, Pipe Handling

Tag description: Crane, Overhead, BOP

-

-

-

Pipe handling Crane

Tag: 362-MV-NNN

Tag: 362-MV-NNN

Tag: 362-MF-NNN

BOP Crane / BOP overhead Crane

X-mas tree overhead crane

Tag description: Crane, Overhead, X-mas Tree

3

63 Monorail systems

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-360-A-AA-NNN (.NN)

Doc: NNN-363-A-AA-NNN (.NN)

NSFI description: Overhead monorail cranes for

lifting/transport operation primarily for use in association with

drilling operations.

Types of asset:

Comment: Function and type code according to Appendix B

MB (Hoists and Trolleys (Monorails))

-

Monorails

Tag: 363-MB-NNN

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

42 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes lifting and transport equipment, primarily for

drilling equipment.

MATERIAL HANDLING EQUIPMENT AND

SYSTEMS

36

3

64 Bop transportation system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-360-A-AA-NNN (.NN)

Doc: NNN-364-A-AA-NNN (.NN)

NSFI description: Cranes, lifting equipment and skids/sliding

doors, etc. primarily for transport and handling of BOP units.

Included are drive units, operational and control equipment and

above surface guidance system for BOP`s in moonpool area.

Comment: Function and type code according to Appendix B

MT (Transportation Carriers/Trolleys/Skids)

Types of asset:

-

-

-

-

-

-

Transportation Carriers/Trolleys/Skids

Tag: 364-MT-NNN

Tag: 364-MN-NNN

Tag: 364-MS-NNN

Tag: 364-MG-NNN

Tag: 364-MO-NNN

Tag: 364-JC-NNN

Rails and racks

MN (Rails and racks)

Stack-up Guides

MS (Stack-up Guides)

Under Hull Guide (UHG)

Over Hull Guiding (OHG)

Remote Control Stands

MG (Under Hull Guide (UHG))

MO (Over Hull Guiding (OHG))

JC (Remote Control Stands)

3

65 Utility hoists

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-360-A-AA-NNN (.NN)

Doc: NNN-365-A-AA-NNN (.NN)

NSFI description: Miscellaneous winches, air tuggers, etc.

which primarily serve main drilling equipment and act as

multipurpose winches/hoists.

Types of asset:

Comment: Function and type code according to Appendix B

LP (Manrider Winches)

-

-

-

Manrider Winches

Tag: 365-LP-NNN

Tag: 365-LU-NNN

Tag: 365-JC-NNN

Tugger Winches

LU (Utility Winches)

Remote Control Stands

JC (Remote Control Stands)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

43 of 95

Group 37 – Service equipment and systems

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes equipment for more specialized operations,

such as cementing, well testing, mud logging, electric well

logging and casing setting etc.

SERVICE EQUIPMENT AND SYSTEMS

37

3

71 Cementing unit and associated system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-370-A-AA-NNN (.NN)

Doc: NNN-371-A-AA-NNN (.NN)

NSFI description: System for mixing, conditioning and transfer

of cement from tank to manifold. Included are surge tanks,

cement mixer w/hopper, ejector, cementing unit, pumps,

chicksan joints, associated piping, fitting and hoses.

Function and type code according to Appendix B

Types of asset:

-

-

-

Cementing Packages / Unit

Tag: 371-BI-NNN

Tag: 371-BS-NNN

BI (Cementing Packages)

Cement Standpipe Manifold

BS (Choke/Production/Injection & Test Manifold)

Blenders/agitators/mixers for Cement mixing

Tag: 371-BY-NNN

Tag: 371-CJ-NNN

Tag: 371-VL-NNN

BY (Hoppers/Mud Guns/Blenders)

CJ (Mixers/Agitators)

VL (Receiver and Surge Drums, Expansion-Head Tanks)

-

-

-

Cement Surge Tank

Cement Dust Cyclone and Collector

Cement piping

Tag: 371-CE-NNN

Tag: 371LNNN

CE (Cyclones)

L (Pipes and lines)

See Section 6.2.1 and 8.1 for pipes and lines

-

Pumps

Tag: 371-PA-NNN

See Appendix B, P for pumps.

See Section 6.1 for Main Equipment and Equipment Packages

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

44 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

SERVICE EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes equipment for more specialized operations,

such as cementing, well testing, mud logging, electric well

logging and casing setting etc.

37

3

72 Well testing equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-370-A-AA-NNN (.NN)

Doc: NNN-372-A-AA-NNN (.NN)

NSFI description: Equipment for lowing tests through drill

string or tubing for evaluation of production potential of a well.

Included are hydraulic latch, test tree, safety valves, slip joints

and/or motion compensator, flow line manifold, DST choke

manifold, LP and HP separator, heater, intermediate storage

tanks w/pumps, burner boom, production burner and burner

control.

Types of assets:

Function and type code according to Appendix B

-

-

-

Well Testing Equipment

Well test Manifold

Tag: 372-QW-NNN

Tag: 372-BS-NNN

Tag: 372-BF-NNN

QW (Test Equipment)

BS (Choke/Production/Injection & Test Manifold)

Burner Booms packages

BF (Mud Burner/Mud Burner Package)

3

3

73 Mud logging equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-370-A-AA-NNN (.NN)

Doc: NNN-373-A-AA-NNN (.NN)

NSFI description: Specialized unit used to identify/analyze

geological samples. Also includes equipment for recording and

analyzing drilling parameters.

Function and type code according to Appendix B

QM (Mud Logging Unit)

Types of assets:

-

Mud logging unit

Tag: 373-QM-NNN

74 Electric well logging equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-370-A-AA-NNN (.NN)

Doc: NNN-374-A-AA-NNN (.NN)

NSFI description: Instruments and special equipment for

downhole measurements and recording. Also including wire line,

lubricator and heave compensator for well logging operations.

Function and type code according to Appendix B

Types of assets:

-

-

Wireline equipment (rig)

Well Logging Unit

Tag: 374-BU-NNN

Tag: 374-QL-NNN

BU (Wireline Equipment)

QL (Well Logging Unit)

`

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

45 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

SERVICE EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

This group includes equipment for more specialized operations,

such as cementing, well testing, mud logging, electric well

logging and casing setting etc.

37

3

75 Special service equipment and supplies

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-370-A-AA-NNN (.NN)

Doc: NNN-375-A-AA-NNN (.NN)

NSFI description: Miscellaneous special service equipment and

supplies not normally provided by rig owner or licensee. Diving

equipment can be quoted as an appropriate example of

equipment within this sub-group.

Types of assets:

Function and type code according to Appendix B

-

-

-

-

Coil Tubing Unit

Tag: 375-QL-NNN

Tag: 375-QD-NNN

Tag: 375-QQ-NNN

Tag: 375-QR-NNN

QL (Well Logging Unit)

Riserless Mud Recovery Packages (RMR)

Workover Control System Equipment (WOCS)

Intervention System Equipment and Tools (ROT)

QD (Riserless Mud Recovery Packages (RMR))

QQ (Workover Control System Equipment (WOCS)

QR (Intervention System Equipment and Tools (ROT))

3

76 Wellhead and tubular equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-370-A-AA-NNN (.NN)

Doc: NNN-376-A-AA-NNN (.NN)

NSFI description: Including casing, casing accessories and

equipment mounted on the seabed for casing hang off, BOP

connection and sub-sea guiding functions during well

establishment. Examples of equipment incorporated are

template, temporary guide frame, permanent guide base, guide

posts, casing hangers, etc.

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

46 of 95

Group 38 – Misc. equipment, systems and services

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

NSFI Description / Tag description / Comments

Miscellaneous equipment, systems and services primarily with

auxiliary functions, and which do not logically belong to any

other main group.

MISC. EQUIPMENT, SYSTEMS AND SERVICES [Main document]

38

3

81 Wellhead and sub-sea running and testing

tools

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-380-A-AA-NNN (.NN)

Doc: NNN-381-A-AA-NNN (.NN)

NSFI description: Miscellaneous running and testing tools used

when setting casing and carrying out other sub-sea operations.

Types of asset:

Function and type code according to Appendix B

-

X-mas Tree System Equipment

Tag: 381-QB-NNN

QB (X-mas Tree System Equipment)

3

82 Sub-Sea inspection system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-380-A-AA-NNN (.NN)

Doc: NNN-381-A-AA-NNN (.NN)

NSFI description: System for inspection and survey of sub-sea

installations and the submerged part of the platform. Included

are TV-monitors, cameras, recorders, projectors, lighting

equipment, cables for signal transmission and power supply,

camera hoists, etc.

Types of asset:

Function and type code according to Appendix B

-

ROV/ROMV and Tools

Tag: 382-QS-NNN

QS (ROV/ROMV and Tools)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

47 of 95

3

DRILLING EQUIPMENT AND SYSTEMS

Doc code / Tag Code

See section 5 and 6

NSFI Description / Tag description / Comments

Miscellaneous equipment, systems and services primarily with

auxiliary functions, and which do not logically belong to any

MISC. EQUIPMENT, SYSTEMS AND SERVICES [Main document]

38

other main group.

3

83 Acoustic position reference system

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-380-A-AA-NNN (.NN)

Doc: NNN-383-A-AA-NNN (.NN)

NSFI description: Whenever installed separately, the acoustic

position reference system should be treated in this sub-group.

Normally including beacon, hydrophones and position indicating

display. This sub-group also includes other drilling operation

assisting equipment based on acoustic signal transmission, such

as marine riser angle indicator, etc. NOTE: If the drilling rig is

equipped with a dynamic positioning system, the acoustic

positions reference system should be treated as an integral part

this and hence designated to Sub-group 408.

Types of asset:

Function and type code according to Appendix B

-

Acoustic Position Reference System

Tag: 383-QA-NNN

QA (Acoustic Position Reference System)

3

84 Miscellaneous instruments and auxiliary

equipment

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-380-A-AA-NNN (.NN)

Doc: NNN-384-A-AA-NNN (.NN)

NSFI description: Including miscellaneous instruments which

have not been referred in the respective sub-groups, e.g.

equipment for remote operation and control and misc. safety

functions. Also including misc. equipment and systems with

auxiliary function(s) during drilling operations, and which do not

logically belong to any other main and/or sub-group. Examples

of the latter are separate cooling system for drilling equipment

and laboratory equipment.

Types of asset:

Function and type code according to Appendix B

-

Riser Instrumentation System (RIS)

Tag: 384-WF-NNN

WF (Riser Instrumentation System (RIS))

3

85 Hydraulic/pneumatic power packs (general

purpose)

Doc: NNN-300-A-AA-NNN (.NN)

Doc: NNN-380-A-AA-NNN (.NN)

Doc: NNN-385-A-AA-NNN (.NN)

NSFI description: Including general purpose

hydraulic/pneumatic power packs for drilling operations, which

have not been referred to any other sub-group. Complete power

packs with power units, hoses, valves etc.

Types of asset:

Function and type code according to Appendix B

-

Hydraulic Power Units (HPU)

Tag: 385-CT-NNN

CT (Hydraulic Power Units (HPU))

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

48 of 95

Main Group 4 - Platform Equipment

Group 40 – Manoeuvring machinery and equipment

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

MANOEUVRING MACHINERY AND

EQUIPMENT

40

4

01 Rudder with welded parts

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-401-A-AA-NNN (.NN)

NSFI description: Including flaps with hinges, or movable

nozzles which act as rudder, with welded-in parts (e.g. nozzles),

including machine work done in the workshops and internal

protection of the rudder.

4

03 Steering gear/-columns, telemotor system,

rudder indicators

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-403-A-AA-NNN (.NN)

NSFI description: Steering plants

Types of asset:

Function and type code according to Appendix B

-

Steering Gear

Tag: 403-DB-NNN

DB (Gearboxes end Couplings)

4

4

06 Brakes

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-406-A-AA-NNN (.NN)

NSFI description: Brakes to stop rig/propulsion plant.

08 Dynamic positioning plant (DP)

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-408-A-AA-NNN (.NN)

NSFI description: System to dynamically maintain rig in

position within a given radius from a point on the sea bed.

Function and type code according to Appendix B

Types of asset:

-

Dynamic Positioning Control System (DP)

Tag: 408-IU-NNN

IU (Dynamic Positioning Control System (DP))

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

49 of 95

Group 41 – Navigation and searching equipment

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

NAVIGATION AND SEARCHING EQUIPMENT

41

4

11 Radar plant

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-411-A-AA-NNN (.NN)

NSFI description: Radar plants for navigation and searching.

Types of asset:

Function and type code according to Appendix B

-

Radar System, Marine and Aviation

Tag: 411-RR-NNN

RR (Radar System, Marine and Aviation)

4

12 Positions indicating systems

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-412-A-AA-NNN (.NN)

NSFI description: Navigation plants.

Types of assets:

Function and type code according to Appendix B

-

Navigation, Positioning and Distance Measuring

Equipment

Satellite Communication System Equipment

Tag: 412-RN-NNN

Tag: 412-RS-NNN

RN (Navigation, Positioning and Distance Measuring Equipment)

RS (Satellite Communication System Equipment)

-

4

4

13 Gyro plants, autopilots, compasses

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-413-A-AA-NNN (.NN)

NSFI description: Navigation/maneuvering equipment.

Types of asset:

Function and type code according to Appendix B

-

Navigation, Positioning and Distance Measuring

Equipment

Tag: 413-RN-NNN

RN (Navigation, Positioning and Distance Measuring Equipment)

14 Underwater searching equipment: Asdic,

echo sounder, speed log

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-414-A-AA-NNN (.NN)

NSFI description: Plants and equipment for underwater

searching and for determination of bottom conditions, water

temperature, depth, speed and distance.

Types of asset:

Function and type code according to Appendix B

-

Navigation, Positioning and Distance Measuring

Equipment

Tag: 414-RN-NNN

RN (Navigation, Positioning and Distance Measuring Equipment)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

50 of 95

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

MANEUVERING MACHINERY AND

EQUIPMENT

41

4

15 Clinometers, trim indicators, load indicators,

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-415-A-AA-NNN (.NN)

NSFI description: Equipment mainly for use in connection with

navigation/maneuvering and loading/discharging.

Function and type code according to Appendix B

etc.

Types of asset:

-

Navigation, Positioning and Distance Measuring

Equipment

Tag: 415-RN-NNN

RN (Navigation, Positioning and Distance Measuring Equipment)

4

4

16 Navigation TV

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-416-A-AA-NNN (.NN)

NSFI description: Television equipment for navigational

purposes, complete with all components, transmission cables etc.

17 Clocks, facsimile recorders and misc. nautical Doc: NNN-400-A-AA-NNN (.NN)

NSFI description: Miscellaneous nautical auxiliary equipment

and other equipment (mainly used in connection with

navigation).

Doc: NNN-417-A-AA-NNN (.NN)

equipment

4

18 Radar, signal, observation and antenna

masts

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-418-A-AA-NNN (.NN)

NSFI description: Radar and signal masts, observation masts

or other masts for support of antennas (aerials) and signal

equipment.

Types of asset:

Function and type code according to Appendix B

RR (Radar System, Marine and Aviation)

-

Radar

Tag: 418-RR-NNN

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

51 of 95

Group 42 – Communication equipment

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

COMMUNICATION EQUIPMENT

42

4

21 Radio Plant

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-420-A-AA-NNN (.NN)

Doc: NNN-421-A-AA-NNN (.NN)

NSFI description: Radio plant for external communication

(main and stand-by installations).

Types of asset:

Function and type code according to Appendix B

RB (SOLAS/GMDSS Radio and General Radio System Equipment)

-

SOLAS/GMDSS Radio and General Radio System

Equipment

Tag: 421-RB-NNN

4

22 Lifeboat Radio Transmitters, Emergency

Radio, Direct. Finder

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-420-A-AA-NNN (.NN)

Doc: NNN-422-A-AA-NNN (.NN)

NSFI description: Communication equipment, primarily for

emergencies.

Types of assets:

Function and type code according to Appendix B

RG (Emergency Telephone)

-

-

Emergency Telephone

Tag: 422-RG-NNN

Tag: 422-RT-NNN

Telemetry System Equipment

RT (Telemetry System Equipment)

4

4

23 Data transmission plants, general purpose

EDB plants

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-420-A-AA-NNN (.NN)

Doc: NNN-423-A-AA-NNN (.NN)

NSFI description: Plant for external data transmission

Types of asset:

Function and type code according to Appendix B

-

Data Communication Network Equipment

Tag: 423-RQ-NNN

RQ (Data Communication Network Equipment)

24 VHF and UHF telephones etc.

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-420-A-AA-NNN (.NN)

Doc: NNN-424-A-AA-NNN (.NN)

NSFI description: Telephone plants, mainly for external

communication, which are not included in 421 and 422.

Types of asset:

Function and type code according to Appendix B

-

UHF/VHF Radio (Onboard)

Tag: 424-RU-NNN

RU (UHF/VHF Radio (Onboard))

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

52 of 95

4

PLATFORM EQUIPMENT

COMMUNICATION EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

42

4

25 Calling systems, command telephone,

telephone plants, walkie-talkie, etc.

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-420-A-AA-NNN (.NN)

Doc: NNN-425-A-AA-NNN (.NN)

NSFI description: Electrical communication systems, mainly for

internal communication.

Types of assets:

Function and type code according to Appendix B

RH (Office Intercommunication System Equipment)

RO (Common Telecom. Equipment)

-

-

-

-

-

Office Intercommunication System Equipment

Common Telecom. Equipment

Tag: 425-RH-NNN

Tag: 425-RO-NNN

Tag: 425-RP-NNN

Tag: 425-RM-NNN

Tag: 425-RA-NNN

PABX System Equipment (include Telephone)

Multiplexer and Network Switch

RP (PABX System Equipment (include Telephone))

RM (Multiplexer and Network Switch)

Public Address and Alarm System

RA (Public Address and Alarm System)

4

27 Light and signal equipment (lanterns,

whistles, etc.)

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-420-A-AA-NNN (.NN)

Doc: NNN-427-A-AA-NNN (.NN)

NSFI description: Light and signal equipment for normal

communication.

Types of asset:

Function and type code according to Appendix B

-

Light and signal equipment (lanterns, whistles, etc.)

Tag: 427-EI-NNN

EI (U-lights and Foghorns)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

53 of 95

Group 43 – Anchoring, mooring and towing equipment

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

A complete automatic thruster assisted mooring system.

ANCHORING MOORING AND TOWING

EQUIPMENT

43

4

31 Anchors with chains and equipment

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-430-A-AA-NNN (.NN)

Doc: NNN-431-A-AA-NNN (.NN)

NSFI description: The platform`s anchor equipment.

Types of assets:

Function and type code according to Appendix B

YM (Anchors)

-

-

Anchors

Tag: 431-YM-NNN

Tag: 431-YL-NNN

Mooring Lines

YL (Mooring Lines)

4

32 Windlasses with chain stoppers, rollers, etc.

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-430-A-AA-NNN (.NN)

Doc: NNN-432-A-AA-NNN (.NN)

NSFI description: Windlasses with steam, hydraulic or

electrical machinery.

Types of assets:

Function and type code according to Appendix B

YW (Anchor and Mooring Winches (Windlasses))

BO (Operator Cabins)

-

-

-

-

Windlasses / Anchor winch

Windlass Control Cabin

Chain Stoppers

Tag: 432-YW-NNN

Tag: 432-BO-NNN

Tag: 432-YG-NNN

Tag: 432-YU-NNN

YG (Chains Stopper)

Chain Rollers

YU (Chain Rollers)

4

33 Combined windlass & mooring winches with

chain stoppers, rollers

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-430-A-AA-NNN (.NN)

Doc: NNN-433-A-AA-NNN (.NN)

NSFI description: Windlasses with extra drums for mooring

lines.

Types of assets:

Function and type code according to Appendix B

YJ (Combined windlass and mooring winches)

BO (Operator Cabins)

-

-

-

-

Combined windlass and mooring winches

Windlass Control Cabin

Chain Stoppers

Tag: 433-YJ-NNN

Tag: 433-BO-NNN

Tag: 433-YG-NNN

Tag: 433-YU-NNN

YG (Chains Stopper)

Chain Rollers

YU (Chain Rollers)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

54 of 95

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

A complete automatic thruster assisted mooring system.

ANCHORING MOORING AND TOWING

EQUIPMENT

43

4

34 Capstans, warping and mooring winches

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-430-A-AA-NNN (.NN)

Doc: NNN-434-A-AA-NNN (.NN)

NSFI description: Winches – vertical or horizontal – for

warping (shifting) and mooring with steam, hydraulic or electrical

machinery.

Types of asset:

Function and type code according to Appendix B

YK (Capstan)

-

Capstan

Tag: 434-YK-NNN

4

35 Fixed mooring equipment

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-430-A-AA-NNN (.NN)

Doc: NNN-435-A-AA-NNN (.NN)

NSFI description: Fixed mooring and warping (shifting)

equipment.

Types of asset:

Function and type code according to Appendix B

-

Fairleads

Tag: 435-YS-NNN

YS (Fairlead)

4

4

36 Loose mooring Equipment

37 Towing equipment

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-430-A-AA-NNN (.NN)

Doc: NNN-436-A-AA-NNN (.NN)

NSFI description: Loose mooring and warping (shifting)

equipment.

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-430-A-AA-NNN (.NN)

Doc: NNN-437-A-AA-NNN (.NN)

NSFI description: Towing equipment for ordinary vessels,

rescue ships, tugs, supply ships and similar.

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

55 of 95

Group 44 – Repair, maintenance and cleaning equipment, outfitting

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

REPAIR, MAINTENANCE AND CLEANING

EQUIPMENT, OUTFITTING

44

4

41 Machine tools, cutting and welding

equipment

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-440-A-AA-NNN (.NN)

Doc: NNN-441-A-AA-NNN (.NN)

NSFI description: Machinery tools (fixed/stationary), cutting

and welding equipment, delivered with the rig for ordinary

maintenance and repair onboard.

Types of assets:

Function and type code according to Appendix B

-

-

Fixed/stationary machinery tools

Cutting and welding equipment

Tag: 441-MH-NNN

Tag: 441-MH-NNN

MH (Workshop Machines)

MH (Workshop Machines)

4

4

4

42 Tools and equipment for engineers and

electricians

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-440-A-AA-NNN (.NN)

Doc: NNN-442-A-AA-NNN (.NN)

NSFI description: Tools and equipment for the engineers,

other hand tools and special tools delivered with the vessel.

43 Painting equipment and scaffolding, paint

rafts/boats (gigs)

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-440-A-AA-NNN (.NN)

Doc: NNN-443-A-AA-NNN (.NN)

NSFI description: Equipment delivered with the vessel for

maintenance painting on board, with associated equipment.

44 Cleaning equipment, garbage chutes, etc.

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-440-A-AA-NNN (.NN)

Doc: NNN-444-A-AA-NNN (.NN)

NSFI description: Equipment delivered with the vessel for

ordinary cleaning on board, except equipment particularly for

galley and laundry.

Types of assets:

Function and type code according to Appendix B

-

-

Central Vacuum Cleaner System Equipment

Tag: 444-AV-NNN

Tag: 444-XJ-NNN

AV (Central Vacuum Cleaner System Equipment)

Vacuum Unit Packages (if not part of the Central

Vacuum Cleaner System)

XJ (Vacuum Units Packages)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

56 of 95

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

REPAIR, MAINTENANCE AND CLEANING

EQUIPMENT, OUTFITTING

44

4

45 Garbage disposal plants

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-440-A-AA-NNN (.NN)

Doc: NNN-445-A-AA-NNN (.NN)

NSFI description: Incinerator for garbage, etc. (or other

garbage disposal device), for rubbish, kitchen refuse, sanitary

system sludge, fuel- and lubricating oil sludge, and possible

other waste, complete with all fixtures and equipment and

associated equipment.

Types of assets:

Function and type code according to Appendix B

CP (Incinerators)

-

-

Incinerator for garbage

Tag: 445-CP-NNN

Tag: 445-CN-NNN

Solid Waste Disposal Units

CN (Solid Waste Disposal Units)

4

46 Outfitting in store rooms and workshops

47 Clamps and foundation for spare parts

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-440-A-AA-NNN (.NN)

Doc: NNN-446-A-AA-NNN (.NN)

NSFI description: Outfitting in workshops and storage rooms

(stores that are not meant for cargo or provisions) and special

rooms not used for living areas, i.e., steering gear compartment,

lamps rooms, compressor room etc.

4

4

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-440-A-AA-NNN (.NN)

Doc: NNN-447-A-AA-NNN (.NN)

NSFI description: Prefabrication and mounting of clamps and

foundation for spare parts and fastening of the spare parts.

48 Name plates (markings) on machinery,

equipment, pipes, cables

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-440-A-AA-NNN (.NN)

Doc: NNN-448-A-AA-NNN (.NN)

NSFI description: Name plates/signs/labels (marking).

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

57 of 95

Group 45 – Lifting and transport equipment for machinery components

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

LIFTING AND TRANSPORT EQUIPMENT FOR

MACHINERY COMPONENTS

45

4

51 Column Lifts

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-450-A-AA-NNN (.NN)

Doc: NNN-451-A-AA-NNN (.NN)

NSFI description: Lifts complete with car (gondola), doors and

machinery.

Types of asset:

Function and type code according to Appendix B

-

Elevators and Lifts

Tag: 451-MC-NNN

MC (Elevators and Lifts)

4

52 Travelling crane and lifting equipment in

machinery rooms

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-450-A-AA-NNN (.NN)

Doc: NNN-452-A-AA-NNN (.NN)

NSFI description: Lifting and transport equipment for

machinery components in engine rooms, pump rooms and other

rooms for machinery.

Types of assets:

Function and type code according to Appendix B

-

-

Monorails above heavy machinery

Travelling / Overhead Cranes

Tag: 452-MB-NNN

Tag: 452-MF-NNN

MB (Hoists and Trolleys (Monorails))

MF (Overhead Cranes)

4

4

54 Mobile transport and handling equipment

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-450-A-AA-NNN (.NN)

Doc: NNN-453-A-AA-NNN (.NN)

NSFI description: Mobile transport and handling equipment for

cement, mud etc. such as fork lifts, pallet trucks, trolleys etc.

with associate charging equipment.

Types of asset:

Function and type code according to Appendix B

-

Trucks, Carts, Forklifts, etc.

Tag: 454-MD-NNN

MD (Trucks, Carts, Forklifts, etc.)

55 Miscellaneous Lifting Appliances (Loose

Equipment)

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-450-A-AA-NNN (.NN)

Doc: NNN-454-A-AA-NNN (.NN)

NSFI description: None

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

58 of 95

Group 48 – Special equipment

4

PLATFORM EQUIPMENT

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

The group includes only the respective special equipment, while

standard equipment (equipment to be found in most platforms)

is included in the respective sub-groups.

SPECIAL EQUIPMENT

48

4

81 Bunker stations

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-480-A-AA-NNN (.NN)

Doc: NNN-481-A-AA-NNN (.NN)

NSFI description: Created by Dolphin Drilling.

Types of asset:

Function and type code according to Appendix B

-

Hose Loading Stations

Tag: 481-MU-NNN

MU (Hose Loading Stations)

4

4

4

83 Diving barge, total

85 Aircraft, helicopters

87 De-icing equipment

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-480-A-AA-NNN (.NN)

Doc: NNN-483-A-AA-NNN (.NN)

NSFI description: Diving equipment.

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-480-A-AA-NNN (.NN)

Doc: NNN-485-A-AA-NNN (.NN)

NSFI description: Aircraft and helicopters equipment.

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-480-A-AA-NNN (.NN)

Doc: NNN-487-A-AA-NNN (.NN)

NSFI description: De-icing equipment for use externally on

drilling equipment, decks, masts, cranes, superstructure, railing,

and other external equipment.

4

89 Derrick skidding systems

Doc: NNN-400-A-AA-NNN (.NN)

Doc: NNN-480-A-AA-NNN (.NN)

Doc: NNN-489-A-AA-NNN (.NN)

NSFI description: Equipment for skidding of derrick, including

such as skidding system with associated pumps, gears with

motors, instrumentation, electrical systems etc. and

locking/clamping mechanism.

Types of asset:

Function and type code according to Appendix B

-

Skid Jacks

Tag: 489-BM-NNN

BM (Skid Jacks)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

59 of 95

Main Group 5 - Equipment for Crew

Group 50 – Lifesaving, protection and medical equipment

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

LIFESAVING, PROTECTION AND MEDICAL

EQUIPMENT

50

5

01 Lifeboats with equipment

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-501-A-AA-NNN (.NN)

NSFI description: Rescue boats (lifeboats) – also combined

lifeboat/workboat – including all equipment on board, loose and

fixed, together with equipment.

Types of assets:

Function and type code according to Appendix B

SA (Lifeboats and Davits)

-

-

Lifeboats

Tag: 501-SA-NNN

Tag: 501-SB-NNN

Man over board (MOB) Pick up boats

SB (Pick-Up Boats and Davits)

5

5

02 Liferafts with equipment

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-502-A-AA-NNN (.NN)

NSFI description: Life rafts with all equipment on board, loose

and fixed, together with raft fasteners and corresponding

equipment as mentioned in sub group 501.

Function and type code according to Appendix B

SW (Rafts)

Types of asset:

-

Liferafts

Tag: 502-SW-NNN

03 Lifesaving, safety and emergency equipment

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-503-A-AA-NNN (.NN)

NSFI description: Lifesaving equipment, safety equipment and

emergency equipment (except lifeboat/rafts).

Types of asset:

Function and type code according to Appendix B

SD (Escape Equipment)

-

-

Escape Equipment

Tag: 503-SD-NNN

Tag: 503-SE-NNN

Life Saving Escape Equipment

SE (Life Saving Escape Equipment)

5

04 Medical and dental equipment, medicines

and first-aid equipment

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-504-A-AA-NNN (.NN)

NSFI description: Equipment of preventative and medical

treatment nature, against injuries and illness.

Types of asset:

Function and type code according to Appendix B

-

First-Aid Equipment

Tag: 504-SC-NNN

SC (First-Aid Equipment)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

60 of 95

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

LIFESAVING, PROTECTION AND MEDICAL

EQUIPMENT

50

5

05 Loose Fire-fighting apparatus and

equipment, firemen’s suit

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-505-A-AA-NNN (.NN)

NSFI description: Fire extinguishers apparatuses and

firefighting equipment, together with firemen`s clothing and

auxiliary equipment.

Types of asset:

Function and type code according to Appendix B

SF (Portable Fire Extinguishers)

-

-

Portable Fire Extinguishers

Mobile Fire Extinguishers

Tag: 505-SF-NNN

Tag: 505-SP-NNN

SP (Mobile Fire Extinguishers)

Group 51 – Insul., panels, bulkheads, doors, side scuttles, windows, skylights

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

INSULATING PANELS, BULKHEADS, DOORS,

SIDE SCUTTLES, WINDOWS, SKYLIGHTS

51

5

11 Insulation, partition bulkheads, panelling,

wallpaper

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-510-A-AA-NNN (.NN)

Doc: NNN-511-A-AA-NNN (.NN)

NSFI description: Insulation against fire, noise and heat, of rig

side, bulkheads, casing, decks, as well as under staircases in

accommodation and around machinery rooms (in all rooms

except freezing/cooling rooms).

5

5

5

12 Doors with coamings, etc., in

accommodation

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-510-A-AA-NNN (.NN)

Doc: NNN-512-A-AA-NNN (.NN)

NSFI description: Internal doors (only purchase/production) in

partition bulkheads/partitions.

14 External doors with coamings, etc.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-510-A-AA-NNN (.NN)

Doc: NNN-514-A-AA-NNN (.NN)

NSFI description: External doors (only purchase/production)

facing open decks.

15 Side scuttles and windows with equipment

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-510-A-AA-NNN (.NN)

Doc: NNN-514-A-AA-NNN (.NN)

NSFI description: All side scuttles, windows and clear-view

fittings and equipment.

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

61 of 95

Group 52 – Internal decks covering, ladders, steps, railing etc.

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

INTERNAL DECK COVERING, LADDERS,

STEPS, RAILING, ETC.

52

5

21 Deck base covering, internal

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-520-A-AA-NNN (.NN)

Doc: NNN-521-A-AA-NNN (.NN)

NSFI description: Base covering on internal decks including

preparation work. If only one type of deck covering is laid, which

also serves as top covering this is included in sub group 522.

5

5

5

22 Deck top covering, internal (linoleum, tiles,

parquet etc.)

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-520-A-AA-NNN (.NN)

Doc: NNN-522-A-AA-NNN (.NN)

NSFI description: Top covering on internal decks with

preparation works as e.g. cleaning, grinding and lacquering.

24 Floor plates (loose), platforms, steps,

ladders, etc. in accommodation

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-520-A-AA-NNN (.NN)

Doc: NNN-524-A-AA-NNN (.NN)

NSFI description: Such outfitting in accommodation.

25 Floor plates (loose), platforms, steps,

ladders, etc. in Pontoon

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-520-A-AA-NNN (.NN)

Doc: NNN-525-A-AA-NNN (.NN)

NSFI description: Such outfitting in pontoons, casing, funnel,

cofferdam, peaks, under forecastle and similar, as well as other

rooms for machinery (boiler rooms, generator rooms, thruster

rooms, steering gear compartment, etc.)

5

5

26 Ladders, platforms, railings, etc. in columns

27 Floors and platforms in machinery spaces

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-520-A-AA-NNN (.NN)

Doc: NNN-526-A-AA-NNN (.NN)

NSFI description: Such outfitting in ballast tanks, fuel tanks,

freshwater tanks and similar.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-520-A-AA-NNN (.NN)

Doc: NNN-527-A-AA-NNN (.NN)

NSFI description: Such outfitting in Machinery spaces

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

62 of 95

Group 53 – External deck covering, steps, ladders etc., fore-and-aft gangway

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

EXTERNAL DECK COVERING, STEPS,

LADDERS, ETC, FORE-AND-AFT GANGWAY

53

5

31 Deck covering, external

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-530-A-AA-NNN (.NN)

Doc: NNN-531-A-AA-NNN (.NN)

NSFI description: Deck covering on external decks and

external platforms including preparation works.

5

32 Wood deck, external

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-530-A-AA-NNN (.NN)

Doc: NNN-531-A-AA-NNN (.NN)

NSFI description: All external wood decks with preparation

work, sanding and similar, and oiling/lacquering, together with

boundary bars and fixing arrangements.

5

5

33 Handrails, railings and rail gates

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-530-A-AA-NNN (.NN)

Doc: NNN-533-A-AA-NNN (.NN)

NSFI description: All external handrails on bulkheads (of steel,

wood, plastic or similar) with fittings, etc. Railings on external

decks, platforms and steps, including stanchions, rails, fittings,

hinges, etc. Rail ports (gates) with fittings and hinges, etc.

34 Ladders, steps, platforms, gratings with

equipment, external

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-530-A-AA-NNN (.NN)

Doc: NNN-534-A-AA-NNN (.NN)

NSFI description: All external steps/stairs, platforms,

grating/grids on external decks and on external

superstructure/deck house with fittings, etc.

5

5

35 Fore-and-aft gangways with equipment

39 Ladders and platforms external on columns

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-530-A-AA-NNN (.NN)

Doc: NNN-535-A-AA-NNN (.NN)

NSFI description: Fore-and-aft gangways (catwalk, storm

bridge) with equipment.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-530-A-AA-NNN (.NN)

Doc: NNN-539-A-AA-NNN (.NN)

NSFI description: Ladders, steps/stairs, platforms,

grating/grids external on columns, complete with fittings.

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

63 of 95

Group 54 – Furniture, inventory and entertainment equipment

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

FURNITURE, INVENTORY AND

ENTERTAINMENT EQUIPMENT

54

5

5

5

5

5

5

41 Furniture for crew, standard furniture

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-540-A-AA-NNN (.NN)

Doc: NNN-541-A-AA-NNN (.NN)

NSFI description: Furniture for crew

42 Office equip. & spec. furniture in

wheelhouse, chartroom, etc.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-540-A-AA-NNN (.NN)

Doc: NNN-542-A-AA-NNN (.NN)

NSFI description: Office equipment, located in offices and

other rooms.

43 Bedclothes and mattresses

44 Curtains with equipment, carpets, etc.

45 Decorations, figureheads

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-540-A-AA-NNN (.NN)

Doc: NNN-543-A-AA-NNN (.NN)

NSFI description: Equipment for all bunks on board such as

mattresses, eiderdowns, pillows, blankets, quilts and bedspreads.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-540-A-AA-NNN (.NN)

Doc: NNN-544-A-AA-NNN (.NN)

NSFI description: All such equipment (furnishing).

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-540-A-AA-NNN (.NN)

Doc: NNN-545-A-AA-NNN (.NN)

NSFI description: Embellishments and decorations whit

mountings.

46 Hobby, sports and entertainment equipment, Doc: NNN-500-A-AA-NNN (.NN)

NSFI description: Equipment for entertainment and leisure

time activities for crew.

Doc: NNN-540-A-AA-NNN (.NN)

Doc: NNN-546-A-AA-NNN (.NN)

etc.

Types of asset:

Function and type code according to Appendix B

-

Entertainment System

Tag: 546-RE-NNN

RE (Entertainment System)

5

47 Inventory, smaller items

Doc: NNN-547-AA-NNN

Drw: NNN-547-A-AA-NNN.(NN)

NSFI description: All small inventory with mountings.

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

64 of 95

Group 55 – Galley & pantry equip., arrangement for provision, ironing/ drying equipment

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

GALLEY & PANTRY EQUIPMENT,

ARRANGEMENT FOR PROVISIONS,

IRONING/DRYING EQUIPMENT

55

5

51 Galley machinery

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-550-A-AA-NNN (.NN)

Doc: NNN-551-A-AA-NNN (.NN)

NSFI description: All galley machinery, complete with

fastening, (usually) placed in galley and/or pantry.

Types of assets:

Function and type code according to Appendix B

AB (Galley equipment)

-

-

All types of galley machinery

Freezer boxes/refrigerators

Tag: 551-AB-NNN

Tag: 551-AB-NNN

AB (Galley equipment)

5

5

52 Equipment/outfitting in galley and pantry

53 Cafeteria plant with equipment

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-550-A-AA-NNN (.NN)

Doc: NNN-552-A-AA-NNN (.NN)

NSFI description: All types of loose galley equipment.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-550-A-AA-NNN (.NN)

Doc: NNN-553-A-AA-NNN (.NN)

NSFI description: Cafeteria counter with heating/refrigeration

machinery (incl. pipes, insulation, etc.), lighting arrangement and

other associated equipment, e.g. such as food containers, trays,

etc.

5

5

54 Freezing and refrigerating systems for

provisions

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-550-A-AA-NNN (.NN)

Doc: NNN-554-A-AA-NNN (.NN)

NSFI description: Freezing/refrigerating system for provision

(supplies) and other goods, (excluding ice cube machines) for

crew.

Types of asset:

Function and type code according to Appendix B

-

Refrigeration and Freezing System Equipment

Tag: 554-AR-NNN

AR (Refrigeration and Freezing System Equipment)

55 Insulation, lining & battening in prov. rooms, Doc: NNN-500-A-AA-NNN (.NN)

NSFI description: Insulating and lining/sheathing in the

provision rooms (also dry-provision rooms).

Doc: NNN-550-A-AA-NNN (.NN)

Doc: NNN-555-A-AA-NNN (.NN)

insul. doors, etc.

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

65 of 95

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

GALLEY & PANTRY EQUIPMENT,

ARRANGEMENT FOR PROVISIONS,

IRONING/DRYING EQUIPMENT

55

5

5

56 Bulkheads, bins, shelves, racks etc. in

provision rooms

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-550-A-AA-NNN (.NN)

Doc: NNN-556-A-AA-NNN (.NN)

NSFI description: Outfitting and inventory in the provision

rooms (refrigerated rand dry-provision rooms).

58 Ironing and drying equipment, laundry

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-550-A-AA-NNN (.NN)

Doc: NNN-558-A-AA-NNN (.NN)

NSFI description: All equipment and machinery with

mountings in laundry/drying room (also in laundry for the crew).

Types of asset:

Function and type code according to Appendix B

-

Laundry washer/drier industrial equipment

Tag: 558-AE-NNN

AE (Laundry Equipment)

Group 56 – Lifting and transport equipment for crew and provisions

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

LIFTING AND TRANSPORT EQUIPMENT FOR

CREW AND PROVISIONS

56

5

61 Personnel lifts, escalators

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-560-A-AA-NNN (.NN)

Doc: NNN-561-A-AA-NNN (.NN)

NSFI description: Lifts (elevators) with doors and cars

(gondolas) and/or escalators for crew and passengers, complete

with machinery.

Types of asset:

Function and type code according to Appendix B

-

Elevators and Lifts

Tag: 561-MC-NNN

MC (Elevators and Lifts)

5

5

64 Shore gangway with davits, etc. landing

booms

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-560-A-AA-NNN (.NN)

Doc: NNN-564-A-AA-NNN (.NN)

NSFI description: Shore gangway and landing equipment.

NSFI description: Landing area for helicopter.

66 Helicopter platform with equipment

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-560-A-AA-NNN (.NN)

Doc: NNN-566-A-AA-NNN (.NN)

Types of asset:

Function and type code according to Appendix B

-

Helicopter refueling system package

Tag: 566-XH-NNN

XH (Helicopter Fuel Packages)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

66 of 95

Group 57 – Ventilation, air-conditioning and heating systems

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

VENTILATION, AIR-CONDITIONING AND

HEATING SYSTEMS

57

5

71 Ventilation/air-conditioning systems for

accommodation, etc.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-570-A-AA-NNN (.NN)

Doc: NNN-571-A-AA-NNN (.NN)

NSFI description: Natural or mechanical ventilation systems or

air-conditioning systems for accommodation (also wheelhouse,

radio room and similar, together with other spaces/rooms served

by the accommodation ventilation system).

Function and type codes according to Appendix B,

See section G - Heating, Ventilation & Air Conditioning (HVAC)

Types of assets not specified.

Tag: 571-GA-NNN

5

72 Ventilation/air-conditioning systems for

provision rooms, etc.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-570-A-AA-NNN (.NN)

Doc: NNN-572-A-AA-NNN (.NN)

NSFI description: Separate ventilation/air-conditioning systems

for provisions rooms (refrigerated and dry-provision rooms)

and/or galley with similar arrangement and equipment as stated

under sub group 571. Ozone plant and humidity regulating plant

for air-conditioning of the above rooms, wind catches/deflectors

for side scuttles and fans, etc.

Function and type codes according to Appendix B,

See section G - Heating, Ventilation & Air Conditioning (HVAC)

Types of assets not specified.

Tag: 572-GA-NNN

5

5

73 Ventilation/air-conditioning systems for

control rooms, etc.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-570-A-AA-NNN (.NN)

Doc: NNN-573-A-AA-NNN (.NN)

NSFI description: Separate ventilation/air-conditioning systems

for control room, workshops in engine room and EDP room, with

similar arrangement and equipment stated in sub group 571.

Function and type codes according to Appendix B,

Types of assets not specified.

Tag: 573-GA-NNN

See section G - Heating, Ventilation & Air Conditioning (HVAC)

74 Ventilation/air-conditioning systems for

machinery spaces

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-570-A-AA-NNN (.NN)

Doc: NNN-574-A-AA-NNN (.NN)

NSFI description: Separate ventilation/air-conditioning systems

for boiler/engine rooms and adjoining rooms for machinery.

Function and type codes according to Appendix B,

See section G - Heating, Ventilation & Air Conditioning (HVAC)

Types of assets not specified.

Tag: 574-GA-NNN

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

67 of 95

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

VENTILATION, AIR-CONDITIONING AND

HEATING SYSTEMS

57

5

75 Ventilation/air-conditioning systems for mud Doc: NNN-500-A-AA-NNN (.NN)

NSFI description: Separate ventilation/air-conditioning systems

for spaces were dangers of gas, explosion, positioning and

similar exist.

Doc: NNN-570-A-AA-NNN (.NN)

Doc: NNN-575-A-AA-NNN (.NN)

rooms, shale shaker room, etc.

Function and type codes according to Appendix B,

See section G - Heating, Ventilation & Air Conditioning (HVAC)

Types of assets not specified.

Tag: 575-GA-NNN

5

5

76 Ventilation/air-conditioning systems for Co2

room, store-rooms, etc.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-570-A-AA-NNN (.NN)

Doc: NNN-576-A-AA-NNN (.NN)

NSFI description: Separate ventilation/air-conditioning systems

for such rooms.

Function and type codes according to Appendix B,

See section G - Heating, Ventilation & Air Conditioning (HVAC)

Types of assets not specified.

Tag: 576-GA-NNN

77 Central heating system with radiators & fan

heaters

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-570-A-AA-NNN (.NN)

Doc: NNN-577-A-AA-NNN (.NN)

NSFI description: Central heating system with radiators or

similar, for all rooms on board, except cargo holds.

Function and type code according to Appendix B,

See section G - Heating, Ventilation & Air Conditioning (HVAC)

Types of asset:

-

-

Boilers and Heaters

Radioators

Tag: 577-GC-NNN

Tag: 577-HR-NNN

GC (Boilers and Heaters)

HR (Radiators)

5

78 Ventilation/air-conditioning systems in

rooms for drilling machinery

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-570-A-AA-NNN (.NN)

Doc: NNN-578-A-AA-NNN (.NN)

NSFI description: Separate ventilation/air-conditioning systems

in rooms for drilling machinery/equipment.

Function and type codes according to Appendix B,

See section G - Heating, Ventilation & Air Conditioning (HVAC)

Types of assets not specified.

Tag: 578-GA-NNN

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

68 of 95

Group 58 – Sanitary systems with discharges

5

EQUIPMENT FOR CREW

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

SANITARY SYSTEMS WITH DISCHARGES

58

5

81 Sanitary supply systems

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-580-A-AA-NNN (.NN)

Doc: NNN-581-A-AA-NNN (.NN)

NSFI description: Sanitary fresh and salt water systems for

supply of hot and cold water.

5

82 Sanitary discharge systems, drainage

systems

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-580-A-AA-NNN (.NN)

Doc: NNN-582-A-AA-NNN (.NN)

NSFI description: Sanitary discharge systems whit air pipes

and drainage systems for the accommodation.

Types of asset:

Function and type code according to Appendix B

-

Sewage/Waste Water Treatment Packages

Tag: 582-XF-NNN

XF (Sewage/Waste Water Treatment Packages)

5

5

83 Bathtubs, bidets, shower cabinets, WC,

washbasins, etc.

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-580-A-AA-NNN (.NN)

Doc: NNN-583-A-AA-NNN (.NN)

NSFI description: Large sanitary components.

Types of asset:

Function and type code according to Appendix B

AC (Sanitary Equipment)

-

Sanitary Equipment

Tag: 583-AC-NNN

84 Drinking water systems and coolers

Doc: NNN-500-A-AA-NNN (.NN)

Doc: NNN-580-A-AA-NNN (.NN)

Doc: NNN-584-A-AA-NNN (.NN)

NSFI description: Separate drinking water systems.

Types of asset:

Function and type code according to Appendix B

-

Potable Water Pump Packages

Tag: 584-XE-NNN

XE (Potable Water Pump Packages)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

69 of 95

Main Group 6 - Machinery Main Components

Group 62 – Other types of propulsion machinery

6

MACHINERY MAIN COMPONENTS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

OTHER TYPES OF PROPULSION MACHINERY

62

6

25 Electric generator - motor plants etc.

Doc: NNN-600-A-AA-NNN (.NN)

Doc: NNN-620-A-AA-NNN (.NN)

Doc: NNN-625-A-AA-NNN (.NN)

NSFI description: Electric generator/electric motor for plants

for propulsion.

Group 63 – Transmission and foils

6

MACHINERY MAIN COMPONENTS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

TRANSMISSIONS AND FOILS

63

6

31 Fixed propeller plant

Doc: NNN-600-A-AA-NNN (.NN)

Doc: NNN-630-A-AA-NNN (.NN)

Doc: NNN-631-A-AA-NNN (.NN)

NSFI description: Propeller plants with one or more fixed

propellers (not counter-rotating or stern/bottom propeller plants)

for propulsion.

6

35 Azimuth Thrusters

Doc: NNN-600-A-AA-NNN (.NN)

Doc: NNN-630-A-AA-NNN (.NN)

Doc: NNN-635-A-AA-NNN (.NN)

NSFI description: Special propeller plants for propulsion

(plants which are not included in sub group 631).

Types of asset:

Function and type code according to Appendix B

-

Thruster Package

Tag: 635-YT-NNN A

YT (Thruster Package)

6

37 Main reduction gears with thrust bearings,

couplings

Doc: NNN-600-A-AA-NNN (.NN)

Doc: NNN-630-A-AA-NNN (.NN)

Doc: NNN-637-A-AA-NNN (.NN)

NSFI description: Reduction gears for propulsion. Couplings

(also clutches and torque converters and sliding valves) for

propulsion.

Types of asset:

Function and type code according to Appendix B

DB (Gearboxes end Couplings)

-

Gearboxes end Couplings

Tag: 637-DB-NNN A

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

70 of 95

Group 64 – Boilers and Steam Plants

6

MACHINERY MAIN COMPONENTS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

BOILERS AND STEAM PLANTS

64

6

44 Boilers, steam and gas generators

Doc: NNN-600-A-AA-NNN (.NN)

Doc: NNN-640-A-AA-NNN (.NN)

Doc: NNN-644-A-AA-NNN (.NN)

NSFI description: None

Types of asset:

Function and type code according to Appendix B

-

Steam Boilers incl. Stack and Ducting

Tag: 644-FB-NNN A

FB (Steam Boilers incl. Stack and Ducting)

Group 65 – Motor aggregates for main electric power production

6

MACHINERY MAIN COMPONENTS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

MOTOR AGGREGATES FOR MAIN ELECTRIC

POWER PRODUCTION

65

6

51 Main Diesel Generator Engines

Doc: NNN-600-A-AA-NNN (.NN)

Doc: NNN-650-A-AA-NNN (.NN)

Doc: NNN-651-A-AA-NNN (.NN)

NSFI description: Auxiliary engines with generators for

electrical power production (motor generators, diesel generating

sets) for main electric power production.

Function and type code according to Appendix B

DD (Diesel Engines Power Production)

Types of asset:

-

-

Diesel Engines

Generators

Tag: 651-DD-NNN A

Tag: 651-EG-NNN A

EG (Generators)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

71 of 95

Group 66 – Other aggregates & generators for main & emergency electrical power production

6

MACHINERY MAIN COMPONENTS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

OTHER AGGREGATES & GENERATORS FOR

MAIN & EMERGENCY ELECTRICAL POWER

PRODUCTION

66

6

65 Emergency Diesel Generator Engines

Doc: NNN-600-A-AA-NNN (.NN)

Doc: NNN-660-A-AA-NNN (.NN)

Doc: NNN-665-A-AA-NNN (.NN)

NSFI description: Engine/turbine with generator for use in

harbors and/or emergency situations. The plant includes

equivalent mentioned in sub group 651.

Types of asset:

Function and type code according to Appendix B

-

Emergency Power Generation Sets

Tag: 665-EO-NNN A

EO (Emergency Power Generation Sets)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

72 of 95

Main Group 7 - Systems for Machinery main Components

Group 70 – Fuel systems

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main fuel systems, i.e.

systems for distribution, treatment and supply of liquid fuel, gas

fuel or solid fuel machinery main components (machinery in main

group 6) and possibly to miscellaneous machinery supplied by

the same systems.

FUEL SYSTEMS

70

7

01 Fuel oil transfer and drain systems

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-701-A-AA-NNN (.NN)

NSFI description: Transfer and drain systems for the drilling

platform`s main fuel oil system.

Function and type code according to Appendix B

PG (Screw Pumps)

Types of assets:

-

-

-

-

-

-

-

Screw Pumps

Tag: 701-PG-NNN

Tag: 701-PF-NNN

Tag: 701-PE-NNN

Tag: 701-DE-NNN

Tag: 701-TB-NNN

Tag: 701-HA-NNN

Tag: 701-HB-NNN

Diaphragm Pumps

PF (Diaphragm Pumps)

Gear Pumps

PE (Gear Pumps)

Electrical Motors

DE (Electrical Motors)

Storage Tanks - Rectangular

Shell and Tube Heat Exchangers

Plate Heat Exchangers

TB (Storage Tanks - Rectangular)

HA (Shell and Tube Heat Exchangers)

HB (Plate Heat Exchangers)

Examples of assets:

Comment: According to Building Specification

PG (Screw Pumps)

-

-

-

-

-

-

-

Fuel Oil Transfer Pumps (Poontoons)

Tag: 701-PG-NNN

Tag: 701-PF-NNN

Tag: 701-PE-NNN

Tag: 701-PE-NNN

Tag: 701-TB-NNN

Tag: 701-HB-NNN

Tag: 701-PG-NNN

Clean Leak Fuel Oil Pumps (Membrane)

Aux. Fuel Oil Transfer Pumps

Clean Fuel Oil Transfer Pumps

Fuel Oil tanks (Settling, service etc.)

Fuel Oil Coolers

PF (Diaphragm Pumps)

PE (Gear Pumps)

PE (Gear Pumps)

TB (Storage Tanks - Rectangular)

HB (Plate Heat Exchangers)

Sludge Pumps

PG (Screw Pumps)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

73 of 95

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main fuel systems, i.e.

systems for distribution, treatment and supply of liquid fuel, gas

fuel or solid fuel machinery main components (machinery in main

group 6) and possibly to miscellaneous machinery supplied by

the same systems.

FUEL SYSTEMS

70

7

02 Fuel oil purification plants

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-702-A-AA-NNN (.NN)

NSFI description: Plants for purification and treatment of

diesel and/or heavy oil (in the drilling platform`s main fuel

system).

Types of asset:

Function and type code according to Appendix B

VA (Separators)

-

Separators

Examples of assets:

Fuel Oil Separator

Tag: 702-VA-NNN

Tag: 702-VA-NNN

Comment: According to Building Specification

-

VA (Separators)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

74 of 95

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main fuel systems, i.e.

systems for distribution, treatment and supply of liquid fuel, gas

fuel or solid fuel machinery main components (machinery in main

group 6) and possibly to miscellaneous machinery supplied by

the same systems.

FUEL SYSTEMS

70

7

03 Fuel oil supply system

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-703-A-AA-NNN (.NN)

NSFI description: Systems for supply of fuel oil (diesel) from

tanks or from branch points in the transfer (main fuel oil) system

to the consumers.

Types of assets:

Function and type code according to Appendix B

CA (Regenerative Filters)

-

-

-

-

-

-

Regenerative Filters

Tag: 703-CA-NNN

Tag: 703-CX-NNN

Tag: 703-PG-NNN

Tag: 703-PF-NNN

Tag: 703-DE-NNN

Tag: 703-TB-NNN

Other Miscellaneous Mechanical Equipment

Screw Pumps

CX (Other Miscellaneous Mechanical Equipment)

PG (Screw Pumps)

Diaphragm Pumps

PF (Diaphragm Pumps)

Electrical Motors

DE (Electrical Motors)

Storage Tanks - Rectangular

TB (Storage Tanks - Rectangular)

Examples of assets:

Comment: According to Building Specification

TB (Storage Tanks - Rectangular)

-

-

-

-

-

-

-

Day Tanks

Tag: 703-TB-NNN

Tag: 703-CA-NNN

Tag: 703-CX-NNN

Tag: 703-PG-NNN

Tag: 703-PG-NNN

Tag: 703-PG-NNN

Tag: 703-PF-NNN

Duplex Filters

CA (Regenerative Filters)

Flow Meters

CX (Other Miscellaneous Mechanical Equipment)

PG (Screw Pumps)

Fuel Oil Booster Pumps

Fuel Oil Utility Service Pumps

Fuel Oil Circulating Pumps

Fuel Oil Pneumatic Pumps (Membrane)

PG (Screw Pumps)

PG (Screw Pumps)

PF (Diaphragm Pumps)

7

04 Heating coils in fuel tanks

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-704-A-AA-NNN (.NN)

NSFI description: Heating coils (heating pipes or equivalent) in

the fuel oil hull tank and possibly in larger loos tanks, e.g.

daytanks (in the main fuel oil system) with connection tanks.

Also included are heating coils in combined fuel oil/ballast tanks.

Function and type code according to Appendix B

Types of asset:

L (Pipes and lines)

See Section 6.2.1 and 8.1 for pipes and lines

-

Piping (only)

Tag: 704LNNN

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

75 of 95

Group 71 – Lube oil systems

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main lube oil system,

i.e. systems for distribution, treatment and supply of lube oil to

machinery main components (machinery in main group 6) and

possibly to miscellaneous machinery supplied by the same

systems.

LUBE OIL SYSTEMS

71

7

11 Lube oil transfer and drain systems

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-710-A-AA-NNN (.NN)

Doc: NNN-711-A-AA-NNN (.NN)

NSFI description: Transfer and drain system for the drilling

platform`s main lubrication oil systems.

Types of assets:

Function and type code according to Appendix B

PG (Screw Pumps)

-

-

-

-

-

Screw Pumps

Tag: 711-PG-NNN

Tag: 711-DE-NNN

Tag: 711-TB-NNN

Tag: 711-HA-NNN

Tag: 711-HB-NNN

Electrical Motors

DE (Electrical Motors)

Storage Tanks - Rectangular

Shell and Tube Heat Exchangers

Plate Heat Exchangers

TB (Storage Tanks - Rectangular)

HA (Shell and Tube Heat Exchangers)

HB (Plate Heat Exchangers)

Examples of assets:

Comment: According to Building Specification

TB (Storage Tanks - Rectangular)

TB (Storage Tanks - Rectangular)

TB (Storage Tanks - Rectangular)

PG (Screw Pumps)

-

-

-

-

Lube Oil Storage Tank

Tag: 711-TB-NNN

Tag: 711-TB-NNN

Tag: 711-TB-NNN

Tag: 711-PG-NNN

LO renovated Tank

Dirty LO renovating Tank

Lube Oil Transfer Pumps

7

12 Lube oil purification plants

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-710-A-AA-NNN (.NN)

Doc: NNN-712-A-AA-NNN (.NN)

NSFI description: Plant for purification and treatment of

lubricating oil (in the drilling platform`s main lube oil system).

Types of assets:

Function and type code according to Appendix B

VA (Separators)

-

Separators

Examples of assets:

Lube Oil Separator

Tag: 712-VA-NNN

Tag: 712-VA-NNN

Comment: According to Building Specification

VA (Separators)

-

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

76 of 95

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main lube oil system,

i.e. systems for distribution, treatment and supply of lube oil to

machinery main components (machinery in main group 6) and

possibly to miscellaneous machinery supplied by the same

systems.

LUBE OIL SYSTEMS

71

7

13 Lube oil system for propulsion machinery

and transmissions

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-710-A-AA-NNN (.NN)

Doc: NNN-713-A-AA-NNN (.NN)

NSFI description: Lubricating oil system for propulsion

machinery and transmission (for machinery in groups 62 and

63).

Types of assets:

Function and type code according to Appendix B

PG (Screw Pumps)

-

-

Screw Pumps

Tag: 713-PG-NNN

Tag: 713-DE-NNN

Electrical Motors

DE (Electrical Motors)

Examples of assets:

Comment: According to Building Specification

PG (Screw Pumps)

-

Lube Oil Pumps

Tag: 713-PG-NNN

7

14 Lube oil systems for motor/turbo

aggregates, etc.

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-710-A-AA-NNN (.NN)

Doc: NNN-714-A-AA-NNN (.NN)

NSFI description: Lubricating oil systems for motor

aggregates, turbo aggregates, etc. (machinery in group 65 and

66).

Types of assets:

Function and type code according to Appendix B

-

-

Screw Pumps

Tag: 714-PG-NNN

Tag: 714-DE-NNN

PG (Screw Pumps)

Electrical Motors

DE (Electrical Motors)

Examples of assets:

Lube Oil Pumps

Comment: According to Building Specification

PG (Screw Pumps)

-

Tag: 714-PG-NNN

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

77 of 95

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main lube oil system,

i.e. systems for distribution, treatment and supply of lube oil to

machinery main components (machinery in main group 6) and

possibly to miscellaneous machinery supplied by the same

systems.

LUBE OIL SYSTEMS

71

7

15 Common lube oil system for other machinery Doc: NNN-700-A-AA-NNN (.NN)

NSFI description: Lubricating oil systems for separate gas

generators for propulsion gas turbines, etc. (machinery in group

Doc: NNN-710-A-AA-NNN (.NN)

Doc: NNN-715-A-AA-NNN (.NN)

64), as well as for machinery not included in main group 6,

which is supplied by the same systems as the stated machinery,

e.g. for bearings in reduction gears and turbines (placed in

engine room) for loading and discharging pumps.

Function and type code according to Appendix B

PG (Screw Pumps)

Types of assets:

-

-

Screw Pumps

Electrical Motors

Tag: 715-PG-NNN

Tag: 715-DE-NNN

DE (Electrical Motors)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

78 of 95

Group 72 – Cooling systems

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main cooling system,

i.e. systems for supply of sea water, fresh water and possibly

other cooling media, to machinery main components (machinery

in main group 6) and possibly to miscellaneous machinery

supplied by the same systems.

COOLING SYSTEM

72

7

21 Sea water cooling system

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-720-A-AA-NNN (.NN)

Doc: NNN-721-A-AA-NNN (.NN)

NSFI description: Main sea water cooling systems for supply to

machinery main components (machinery in main group 6) and to

miscellaneous machinery/equipment.

Function and type code according to Appendix B

PA (Centrifugal Pumps)

Types of assets:

-

-

Centrifugal Pumps

Electrical Motors

Tag: 721-PA-NNN

Tag: 721-DE-NNN

DE (Electrical Motors)

Examples of assets:

Comment: According to Building Specification

PA (Centrifugal Pumps)

-

-

-

-

-

-

-

Seawater Cooling Pumps for ME

Tag: 721-PA-NNN

Tag: 721-PA-NNN

Tag: 721-PA-NNN

Tag: 721-PA-NNN

Tag: 721-PA-NNN

Seawater Cooling Pumps for Auxiliaries

Seawater Cooling Pumps for Fresh Water Generator

Seawater Service Pumps

PA (Centrifugal Pumps)

PA (Centrifugal Pumps)

PA (Centrifugal Pumps)

Thruster Seawater Cooling Pumps

PA (Centrifugal Pumps)

Sea Water Pump for Burner Booms & Hull Side Deluge Tag: 721-PA-NNN

Sea Water Utility Pump Tag: 721-PA-NNN

PA (Centrifugal Pumps)

PA (Centrifugal Pumps)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

79 of 95

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main cooling system,

i.e. systems for supply of sea water, fresh water and possibly

other cooling media, to machinery main components (machinery

in main group 6) and possibly to miscellaneous machinery

supplied by the same systems.

COOLING SYSTEM

72

7

22 Fresh water and other cooling Systems

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-720-A-AA-NNN (.NN)

Doc: NNN-722-A-AA-NNN (.NN)

NSFI description: Main fresh water cooling systems with other

media (sea water) for supply to machinery main components

(machinery in main group 6) and to miscellaneous

machinery/equipment.

Types of assets:

Function and type code according to Appendix B

PA (Centrifugal Pumps)

-

-

-

-

Centrifugal Pumps

Tag: 722-PA-NNN

Tag: 722-DE-NNN

Tag: 722-HB-NNN

Tag: 722-VL-NNN

Electrical Motors

DE (Electrical Motors)

Plate Heat Exchangers

HB (Plate Heat Exchangers)

Receiver and Surge Drums, Expansion-Head Tanks

VL (Receiver and Surge Drums, Expansion-Head Tanks)

Comment: According to Building Specification

PA (Centrifugal Pumps)

Examples of assets:

-

-

-

-

-

-

-

Freshwater Cooling Pumps for ME

Tag: 722-PA-NNN

Tag: 722-PA-NNN

Tag: 722-PA-NNN

Tag: 722-HB-NNN

Tag: 722-HB-NNN

Tag: 722-HB-NNN

Tag: 722-VL-NNN

Freshwater Cooling Pumps for Utilities

Thruster Freshwater Pumps

Central Cooler

PA (Centrifugal Pumps)

PA (Centrifugal Pumps)

HB (Plate Heat Exchangers)

Utility Cooler

HB (Plate Heat Exchangers)

Thruster Cooler

HB (Plate Heat Exchangers)

Chemical Filling & Expansion Tanks

VL (Receiver and Surge Drums, Expansion-Head Tanks)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

80 of 95

Group 73 – Compressed air systems

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main compressed air

systems, i.e. systems for production, storage and supply of

compressed air to machinery main components (machinery in

main group 6) and to other machinery supplied by the same

systems, as well as supply of general purposes and instrument

air.

COMPRESSED AIR SYSTEMS

73

7

31 Starting air systems (high pressure)

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-730-A-AA-NNN (.NN)

Doc: NNN-731-A-AA-NNN (.NN)

NSFI description: System for production, storage and supply

of starting and maneuvering air (high-pressure air).

Types of assets:

Function and type code according to Appendix B

KB (Reciprocating Compressors)

-

-

-

Reciprocating Compressors

Tag: 731-KB-NNN

Tag: 731-VK-NNN

Tag: 731-VL-NNN

Dryers

VK (Dryers)

Receiver

VL (Receiver and Surge Drums, Expansion-Head Tanks)

7

33 General purpose air system (low pressure)

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-730-A-AA-NNN (.NN)

Doc: NNN-733-A-AA-NNN (.NN)

NSFI description: General purpose air system (low-pressure

system) for production, storage and supply of compressed air to

taps and equipment on weather decks, on mast

houses/superstructure and in possible other rooms outside the

engine room.

Types of assets:

Function and type code according to Appendix B

KB (Reciprocating Compressors)

-

-

-

Reciprocating Compressors

Tag: 733-KB-NNN

Tag: 733-VK-NNN

Tag: 733-VL-NNN

Dryers

VK (Dryers)

Receiver

VL (Receiver and Surge Drums, Expansion-Head Tanks)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

81 of 95

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main compressed air

systems, i.e. systems for production, storage and supply of

compressed air to machinery main components (machinery in

main group 6) and to other machinery supplied by the same

systems, as well as supply of general purposes and instrument

air.

COMPRESSED AIR SYSTEMS

73

7

34 Instrument air supply system

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-730-A-AA-NNN (.NN)

Doc: NNN-734-A-AA-NNN (.NN)

NSFI description: Supply system, as well as possible plant for

production and storage of compressed air, to instruments and

regulating devices (air which is usually dried and purified). The

instrument air system (supply air system, control air system).

Function and type code according to Appendix B

Types of assets:

-

-

-

Instrument Air Package

Dryers

Tag: 734-IM-NNN

Tag: 734-VK-NNN

Tag: 734-VL-NNN

IM (Instrument Air Package)

VK (Dryers)

Receiver

VL (Receiver and Surge Drums, Expansion-Head Tanks)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

82 of 95

Group 74 – Exhaust systems and air intakes

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes the drilling platform`s main exhaust gas and

air intake systems, i.e. exhaust and air intake systems for

machinery main components (machinery in main group 6).

NSFI description: Fresh air intakes from open air/top casing to

machinery main components (machinery in main group 6) such

as diesel engines, gas turbines and boilers.

EXHAUST SYSTEM AND AIR INTAKES

74

7

7

7

41 Fresh air intakes (not ventilation)

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-740-A-AA-NNN (.NN)

Doc: NNN-741-A-AA-NNN (.NN)

44 Exhaust gas systems for diesel generator

engines, etc.

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-740-A-AA-NNN (.NN)

Doc: NNN-744-A-AA-NNN (.NN)

NSFI description: Exhaust gas system for machinery main

components in group 65 and 66 (electric power producing

machinery).

45 Exhaust gas system for boilers, etc.

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-740-A-AA-NNN (.NN)

Doc: NNN-745-A-AA-NNN (.NN)

NSFI description: Exhaust gas system for machinery main

components in group 64 (boilers, etc.).

Group 76 – Distilled and make-up water systems

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

DISTILLED AND MAKE UP WATER SYSTEMS

76

7

61 Distilled and make-up water systems

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-760-A-AA-NNN (.NN)

Doc: NNN-761-A-AA-NNN (.NN)

NSFI description: Distilled and make-up water systems.

Types of assets:

Function and type code according to Appendix B

XD (Fresh Water Maker Package)

-

Fresh Water Maker Package

Examples of assets:

Fresh Water Generator

Tag: 761-XD-NNN

Tag: 761-XD-NNN

Comment: According to Building Specification

XD (Fresh Water Maker Package)

-

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

83 of 95

Group 79 – Automation systems for machinery

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

See section 5 and 6

[Main document]

The group includes equipment and systems of electrical,

electronic, pneumatic and/or hydraulic types or combination of

these.

AUTOMATION SYSTEMS FOR MACHINERY

79

7

91 Manoeuvre consoles, main consoles

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-790-A-AA-NNN (.NN)

Doc: NNN-791-A-AA-NNN (.NN)

NSFI description: Maneuvering consoles (control consoles) and

main consoles/panels (common cabinets, panel or similar for

mounting of automation equipment belonging to more than one

sub group) in control room/engine room and on bridge, for

installation of automation components for machinery in engine

room and for other machinery in main group 6, 7 and possible 8,

as well as for installation of possible navigation and/or

communication equipment.

Types of assets:

Function and type code according to Appendix B

-

Desks/Consoles, Operator Stations, Work Stat., PC,

VDU

Tag: 791-IE-NNN

IE (Desks/Consoles, Operator Stations, Work Stat., PC, VDU)

7

7

92 Common automation equipment (engine

room alarm, etc.)

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-790-A-AA-NNN (.NN)

Doc: NNN-792-A-AA-NNN (.NN)

NSFI description: Common automation equipment for sub

group in 79, including equipment.

Types of assets:

Function and type code according to Appendix B

-

Control Panels with Instrument, Controllers, etc.

Tag: 792-IB-NNN

IB (Control Panels with Instrument, Controllers, etc.)

93 Automation equipment for propulsion

machinery and transmissions

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-790-A-AA-NNN (.NN)

Doc: NNN-793-A-AA-NNN (.NN)

NSFI description: Automation equipment for propulsion

machinery and transmission (for machinery in groups 62 and

63).

Types of assets:

Function and type code according to Appendix B

-

Control Panels with Instrument, Controllers, etc.

Tag: 793-IB-NNN

IB (Control Panels with Instrument, Controllers, etc.)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

84 of 95

7

SC YO SMTPEOMNS E FNOTRS MACHINERY MAIN

Doc code / Tag Code

NSFI Description / Tag description / Comments

The group includes equipment and systems of electrical,

electronic, pneumatic and/or hydraulic types or combination of

these.

See section 5 and 6

[Main document]

AUTOMATION SYSTEMS FOR MACHINERY

79

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-790-A-AA-NNN (.NN)

Doc: NNN-794-A-AA-NNN (.NN)

NSFI description: Automation equipment for main boilers,

large auxiliary boilers etc. including boiler and burner automation

equipment (for machinery in group 64).

7

94 Automation equipment for boilers, etc.

7

95 Automation equipment for diesel generator

engines etc.

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-790-A-AA-NNN (.NN)

Doc: NNN-795-A-AA-NNN (.NN)

NSFI description: Automation equipment for motor

aggregates, turbo aggregates, etc. (for machinery in group 65).

Types of assets:

Function and type code according to Appendix B

-

Control Panels with Instrument, Controllers, etc.

Tag: 795-IB-NNN

IB (Control Panels with Instrument, Controllers, etc.)

7

7

96 Power management system

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-790-A-AA-NNN (.NN)

Doc: NNN-796-A-AA-NNN (.NN)

NSFI description:

97 Automation equipment for other machinery

components

Doc: NNN-700-A-AA-NNN (.NN)

Doc: NNN-790-A-AA-NNN (.NN)

Doc: NNN-797-A-AA-NNN (.NN)

NSFI description: Automation equipment for other machinery

components; other machinery in engine rooms, except the

machinery mentioned in the foregoing sub groups in group 79,

for systems in engine room and otherwise in main group 7 and

8, unless otherwise stated.

Types of assets:

Function and type code according to Appendix B

-

Control Panels with Instrument, Controllers, etc.

Tag: 796-IB-NNN

IB (Control Panels with Instrument, Controllers, etc.)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

85 of 95

Main Group 8 - Platform Common Systems

Group 80 – Ballast and bilge systems, gutter pipes outside accommodation

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

BALLAST AND BILGE SYSTEMS, GUTTER

PIPES OUTSIDE ACCOMMODATION

80

8

01 Ballast Systems, solid ballast

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-801-A-AA-NNN (.NN)

NSFI description: Systems for water filling/emptying of all

ballast tanks and/or combined ballast/fuel oil tanks.

Types of assets:

Function and type code according to Appendix B

PA (Centrifugal Pumps)

-

-

-

-

-

Centrifugal Pumps

Tag: 801-PA-NNN

Tag: 801-PI-NNN

Tag: 801-CQ-NNN

Tag: 801-DE-NNN

Tag: 801-CT-NNN

Submerged Pumps with Submerged Motors

PI (Submerged Pumps with Submerged Motors)

CQ (Eductors and Ejectors)

Ejector

Electrical Motors

HPU

DE (Electrical Motors)

CT (Hydraulic Units)

Examples of assets:

Comment: According to Building Specification

PA (Centrifugal Pumps)

-

-

-

Ballast Pumps

Tag: 801-PA-NNN

Tag: 801-CQ-NNN

Tag: 801-CT-NNN

Ballast Stripping Ejectors

CQ (Eductors and Ejectors)

HPU for remote control of valves

CT (Hydraulic Units)

8

02 Heating coils in ballast tanks

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-802-A-AA-NNN (.NN)

NSFI description: Heating coils in ballast tanks, including

connection pipes.

Types of assets:

Function and type code according to Appendix B

L (Pipes and lines)

See Section 6.2.1 and 8.1 for pipes and lines

-

Piping (only)

Tag: 802LNNN

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

86 of 95

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

BALLAST AND BILGE SYSTEMS, GUTTER

PIPES OUTSIDE ACCOMMODATION

80

8

03 Bilge systems

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-803-A-AA-NNN (.NN)

NSFI description: System for removal of bilge-water from

pontoons, columns and upper deck.

Types of asset:

Function and type code according to Appendix B

XA (Oily Water Treatment Packages)

PA (Centrifugal Pumps)

-

-

-

-

-

-

-

Oily Water Treatment Packages

Centrifugal Pumps

Tag: 803-XA-NNN

Tag: 803-PA-NNN

Tag: 803-PB-NNN

Tag: 803-PF-NNN

Tag: 803-PH-NNN

Tag: 803-CQ-NNN

Tag: 803-DE-NNN

Reciprocating Pumps

Diaphragm Pumps

PB (Reciprocating Pumps)

PF (Diaphragm Pumps)

Submerged Pumps with Non-Submerged Motors

Ejector

PH (Submerged Pumps with Non-Submerged Motors)

CQ (Eductors and Ejectors)

Electrical Motors

DE (Electrical Motors)

Examples of assets:

Comment: According to Building Specification

PA (Centrifugal Pumps)

-

-

-

-

Main Bilge Pumps (pontoons)

Tag: 803-PA-NNN

Tag: 803-PB-NNN

Tag: 803-PA-NNN

Tag: 803-CQ-NNN

Auxiliary Bilge Pumps (pontoons)

Deck Bilge Pumps

PB (Reciprocating Pumps)

PA (Centrifugal Pumps)

Ejectors, Chain Lockers

CQ (Eductors and Ejectors)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

87 of 95

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

BALLAST AND BILGE SYSTEMS, GUTTER

PIPES OUTSIDE ACCOMMODATION

80

8

04 Gutter pipes outside accommodation

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-804-A-AA-NNN (.NN)

NSFI description: Gutter pipes (drain pipes/scuppers) outside

accommodation from weather decks (also from awnings) and

decks.

Types of asset:

Function and type code according to Appendix B

PB (Reciprocating Pumps)

-

-

Reciprocating Pumps

Electrical Motors

Tag: 804-PB-NNN

Tag: 804-DE-NNN

DE (Electrical Motors)

Examples of assets:

Comment: According to Building Specification

PB (Reciprocating Pumps)

-

-

-

-

Transfer Pumps

Tag: 804-PB-NNN

Tag: 804-PB-NNN

Tag: 804-PB-NNN

Tag: 804-PB-NNN

Slop Water Treatment Pumps

Slop Water Return Pumps

Waste Oil Transfer Pumps

PB (Reciprocating Pumps)

PB (Reciprocating Pumps)

PB (Reciprocating Pumps)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

88 of 95

Group 81 – Fire & lifeboat alarm, fire- fighting & wash down systems

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

NSFI Description / Tag description / Comments

FIRE & LIFEBOAT ALARM, FIRE-FIGHTING- & [Main document]

WASH DOWN SYSTEMS

81

8

11 Fire detection, fire and lifeboat alarm

systems

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-810-A-AA-NNN (.NN)

Doc: NNN-811-A-AA-NNN (.NN)

NSFI description: System for fire detection, fire and lifeboat

alarm systems (general alarm).

Types of asset:

Function and type code according to Appendix B

IB (Control Panels with Instrument, Controllers, etc.)

IF (Field Termination and Cross Connection Cabinets)

SM (Fire Water System Equipment)

-

-

-

Control Panels with Instrument, Controllers, etc.

Field Termination and Cross Connection Cabinets

Fire Water System Equipment

Tag: 811-IB-NNN

Tag: 811-IF-NNN

Tag: 811-SM-NNN

8

12 Emergency shutdown system (ESD)

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-810-A-AA-NNN (.NN)

Doc: NNN-812-A-AA-NNN (.NN)

NSFI description: None

Types of asset:

Function and type code according to Appendix B

IW (Hydraulic Emergency Shut-Down System Equipment)

EQ (Uninterruptible Power Supply (UPS))

-

-

-

-

Hydraulic Emergency Shut-Down System Equipment

Uninterruptible Power Supply (UPS)

Tag: 812-IW-NNN

Tag: 812-EQ-NNN

Tag: 812-IB-NNN

Tag: 812-IF-NNN

Control Panels with Instrument, Controllers, etc.

Field Termination and Cross Connection Cabinets

IB (Control Panels with Instrument, Controllers, etc.)

IF (Field Termination and Cross Connection Cabinets)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

89 of 95

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

NSFI Description / Tag description / Comments

FIRE & LIFEBOAT ALARM, FIRE-FIGHTING- & [Main document]

WASH DOWN SYSTEMS

81

8

13 Fire/wash down systems, emergency fire

pumps, general service pumps

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-810-A-AA-NNN (.NN)

Doc: NNN-813-A-AA-NNN (.NN)

NSFI description: Systems for supply of fire extinguishers

and/or wash down water.

Types of assets:

Function and type code according to Appendix B

PJ (Fire Water Pump Package)

PA (Centrifugal Pumps)

-

-

-

-

-

-

Fire Water Pump Package

Centrifugal Pumps

Tag: 813-PJ-NNN

Tag: 813-PA-NNN

Tag: 813-DE-NNN

Tag: 813-SK-NNN

Tag: 813-SN-NNN

Tag: 813-SS-NNN

Electrical Motors

DE (Electrical Motors)

Dual Agent Unit

SK (Dual Agent Unit)

Remote Operated Foam Monitors

Fire Water Monitors (Towers)

SN (Remote Operated Foam Monitors)

SS (Fire Water Monitors (Towers))

Comment: According to Building Specification

PA (Centrifugal Pumps)

Examples of assets:

-

-

Fire / Deluge Pumps

“Jockey” Pumps for Fire Main

Tag: 813-PA-NNN

Tag: 813-PA-NNN

PA (Centrifugal Pumps)

8

14 Fire-fighting systems for external fires

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-810-A-AA-NNN (.NN)

Doc: NNN-814-A-AA-NNN (.NN)

NSFI description: Systems primarily for extinguishers of

external fires (fires outside the platform), for fires onboard rig,

the other sub groups in 81, to be used.

8

8

15 Fixed fire-fighting system with CO2

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-810-A-AA-NNN (.NN)

Doc: NNN-815-A-AA-NNN (.NN)

NSFI description: Plant/systems for fire-fighting with C02

gases.

16 Fire-fighting systems with foam

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-810-A-AA-NNN (.NN)

Doc: NNN-816-A-AA-NNN (.NN)

NSFI description: Stationary or portable plants (systems) for

foam fire-fighting.

Types of assets:

Function and type code according to Appendix B

-

Fixed Foam Unit

Tag: 816-SJ-NNN

SJ (Fixed Foam Unit)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

90 of 95

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

NSFI Description / Tag description / Comments

FIRE & LIFEBOAT ALARM, FIRE-FIGHTING- & [Main document]

WASH DOWN SYSTEMS

81

8

17 Fixed fire-fighting system with inergen /

water mist system or similar

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-810-A-AA-NNN (.NN)

Doc: NNN-817-A-AA-NNN (.NN)

NSFI description: Fixed plants (systems) for fire-fighting.

Types of assets:

Function and type code according to Appendix B

SH (Inert Gas Equipment)

-

-

-

Inert Gas Equipment

Tag: 817-SH-NNN

Tag: 817-SI-NNN

Tag: 817-SL-NNN

Inert Gas Generator Package

Dry Chemical Equipment

SI (Inert Gas Generator Package)

SL (Dry Chemical Equipment)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

91 of 95

Group 82 – Air and sounding systems from tanks to deck

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

AIR AND SOUNDING SYSTEMS FROM TANKS

TO DECK

82

8

21 Air and sounding systems from tanks to deck Doc: NNN-800-A-AA-NNN (.NN)

NSFI description: Air and sounding systems from tanks to deck

(also combined air- and overflow pipes), for tanks for water, fuel

oil, lube oil, etc. as well as for bilges and special rooms, e.g.

pontoons. Leak Detection System for pontoons and columns.

Doc: NNN-820-A-AA-NNN (.NN)

Doc: NNN-821-A-AA-NNN (.NN)

Group 83 – Special common hydraulic oil systems

8

Platform Common Systems

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

SPECIAL COMMON HYDRAULIC OIL

SYSTEMS

83

8

31 Special common hydraulic oil systems

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-830-A-AA-NNN (.NN)

Doc: NNN-831-A-AA-NNN (.NN)

NSFI description: Hydraulic systems which serve special

consumer combinations, e.g., hydraulic oil systems for

machinery/equipment in more than one sub group and which

cannot be entered in other sub groups for frequent occurring

common hydraulic oil systems.

Types of assets:

Function and type code according to Appendix B

-

Special/Common Hydraulic Power Units

Tag: 831-CT-NNN

CT (Hydraulic Power Units (HPU))

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

92 of 95

Group 84 – Central heat transfer systems with chemical liquids

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

The group includes central heat transfer systems with chemical

liquids (e.g. so-called heat oil/thermal oil system) primarily for

heating of the rig`s tanks, systems, accommodation, etc. If the

heat oil system is also used (partly) for cooling, this is posted

here.

CENTRAL HEAT TRANSFER SYSTEMS WITH

CHEMICAL LIQUIDS

84

8

8

41 Heating oil treatment system

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-840-A-AA-NNN (.NN)

Doc: NNN-841-A-AA-NNN (.NN)

NSFI description: System for treatment and heating of the

chemical heat transfer liquid. This sub group covers the part of

heat oil system that is between boiler flanges for delivery- and

return lines from the users.

42 Heating oil distribution systems for engine

room

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-840-A-AA-NNN (.NN)

Doc: NNN-842-A-AA-NNN (.NN)

NSFI description: System for distribution of chemical heat

transfer liquid to users in engine room and in adjoining rooms.

Group 85 – Automation common systems

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

AUTOMATION COMMON SYSTEMS

85

8

8

8

8

51 Integrated automation system (IAS)

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-850-A-AA-NNN (.NN)

Doc: NNN-851-A-AA-NNN (.NN)

NSFI description: None.

NSFI description: None.

NSFI description: None.

NSFI description: None.

52 Automation system for watertight integrity

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-850-A-AA-NNN (.NN)

Doc: NNN-852-A-AA-NNN (.NN)

53 Automation system for ventilation-, air-

conditioning- and heating systems

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-850-A-AA-NNN (.NN)

Doc: NNN-853-A-AA-NNN (.NN)

54 Automation system for fire-fighting- & wash

down systems

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-850-A-AA-NNN (.NN)

Doc: NNN-854-A-AA-NNN (.NN)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

93 of 95

Group 86 – Electrical power supply

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

ELECTRICAL POWER SUPPLY

86

8

61 Generators/alternators

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-860-A-AA-NNN (.NN)

Doc: NNN-861-A-AA-NNN (.NN)

NSFI description: Generators for main and emergency

electrical power production.

Types of assets:

Function and type code according to Appendix B

-

Generators

Tag: 861-EG-NNN

EG (Generators)

8

65 Transformers

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-860-A-AA-NNN (.NN)

Doc: NNN-865-A-AA-NNN (.NN)

NSFI description: Equipment for electrical conversion,

connected to the drilling platform`s main electrical distribution

plant.

Types of assets:

Function and type code according to Appendix B

EF (Emergency Transformers)

ET (Transformers)

-

-

Emergency Transformers

Main Transformers

Tag: 865-EF-NNN

Tag: 865-ET-NNN

8

8

66 Batteries and chargers

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-860-A-AA-NNN (.NN)

Doc: NNN-866-A-AA-NNN (.NN)

NSFI description: Equipment connected to the drilling

platform`s main electrical distribution plant.

Types of assets:

Function and type code according to Appendix B

-

Batteries

Tag: 866-BA-NNN

BA (Batteries)

67 Converters and rectifiers

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-860-A-AA-NNN (.NN)

Doc: NNN-867-A-AA-NNN (.NN)

NSFI description: Equipment for electrical conversion,

connected to the drilling platform`s main electrical distribution

plant.

Types of assets:

Function and type code according to Appendix B

ER (Rectifiers, inverters, and frequency converters)

-

Rectifiers, inverters, and frequency converters

Tag: 867-ER-NNN

8

68 Shore supply

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-860-A-AA-NNN (.NN)

Doc: NNN-868-A-AA-NNN (.NN)

NSFI description: Electrical systems for supply to main

distributors (switchboard mentioned in sub group 871).

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

94 of 95

Group 87 – Electrical distribution common systems

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

ELECTRICAL DISTRIBUTION COMMON

SYSTEMS

87

8

71 Main switchboard

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-870-A-AA-NNN (.NN)

Doc: NNN-871-A-AA-NNN (.NN)

NSFI description: Main switchboards, included all installation

work as well as installed equipment, such as rails, clamps,

instruments, selectors, switches (also generator circuit breakers

and generator protection equipment), lights, as well as manual

magnetizing, synchronizing, phasing-in and load sharing

equipment.

Types of assets:

Function and type code according to Appendix B

Switch-gear / distribution boards above 1 k V

Switch-gear / distribution boards ; U less or equal to 400V

Tag: 871-EH-NNN

Tag: 871-EL-NNN

EH (Switch-gear / distribution boards above 1 k V)

EL (Switch-gear / distribution boards ; U less or equal to 400V)

Switch-gear / distribution boards ; U above 400 V , U less or

equal to 1 kV

Tag: 871-EN-NNN

EN (Switch-gear / distribution boards ; U above 400 V , U less or equal to 1 kV)

8

72 Emergency switchboard

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-870-A-AA-NNN (.NN)

Doc: NNN-872-A-AA-NNN (.NN)

NSFI description: Emergency switchboard included all

installation work as well as installed equipment, such as rails,

clamps, instruments, selectors, switches (also generator circuit

breakers and generator protection equipment), lights, as well as

manual magnetizing, synchronizing, phasing-in and load sharing

equipment.

Types of assets:

Function and type code according to Appendix B

Switch-gear / distribution boards above 1 k V

Switch-gear / distribution boards ; U less or equal to 400V

Tag: 872-EH-NNN

Tag: 872-EL-NNN

EH (Switch-gear / distribution boards above 1 k V)

EL (Switch-gear / distribution boards ; U less or equal to 400V)

Switch-gear / distribution boards ; U above 400 V , U less or

equal to 1 kV

Tag: 872-EN-NNN

EN (Switch-gear / distribution boards ; U above 400 V , U less or equal to 1 kV)

Doc. ID (rev.)

T-MA-10672(3)

Document Title

Page

Appendix A - Engineering Numbering System Manual

95 of 95

8

PLATFORM COMMON SYSTEMS

Doc code / Tag Code

See section 5 and 6

[Main document]

NSFI Description / Tag description / Comments

ELECTRICAL DISTRIBUTION COMMON

SYSTEMS

87

8

73 Group starters

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-870-A-AA-NNN (.NN)

Doc: NNN-873-A-AA-NNN (.NN)

NSFI description: Group starters, included all installation work,

as well as installed equipment, such as rails, clamps,

instruments, selectors, switches, lights, as well as manual

magnetizing, synchronizing, phasing-in and load sharing

equipment.

8

8

74 Local starters

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-870-A-AA-NNN (.NN)

Doc: NNN-874-A-AA-NNN (.NN)

NSFI description: Local starters, included all installation work,

as well as installed equipment, such as rails, clamps,

instruments, selectors, switches, lights etc.

75 Distribution panels and boards

Doc: NNN-800-A-AA-NNN (.NN)

Doc: NNN-870-A-AA-NNN (.NN)

Doc: NNN-875-A-AA-NNN (.NN)

NSFI description: Distribution panels and boards, including all

installations work, as well as installed equipment, such as rails,

clamps, instruments, selectors, switches, lights etc.

Function and type code according to Appendix B

ED (DC Distribution Boards)

Types of asset:

-

-

-

-

DC Distribution Boards

Tag: 875-ED-NNN

Tag: 875-EE-NNN

Tag: 875-EM-NNN

Tag: 875-EQ-NNN

Emergency Distribution Panels

Main Distribution Panels

UPS Distribution Boards

EE (Emergency Distribution Panels)

EM (Main Distribution Panels)

EQ (Uninterruptible Power Supply (UPS))